## Antigua/Barbuda Hazard Vulnerability Assessment Project: Final Report

Post-Georges Disaster Mitigation Project

in Antigua & Barbuda and St. Kitts & Nevis

May 2001

### Post-Georges Disaster Mitigation in Antigua & Barbuda and St. Kitts & Nevis is implemented by the Organization of American States, Unit for Sustainable Development and Environment for USAID-Jamaica/Caribbean Regional Program

Organization of American States

Unit of Sustainable Development and Environment

1889 F Street NW Washington DC 20006

http://www.oas.org/pgdm

## This report was prepared under contract with the OAS by Eva Hodgkinson-Chin, MSc. [gce@trinidad.net]

# Table of Contents

# Part I – Project and Methodology						Page

# 1.0	Introduction									1

1.1	Post Georges Disaster Mitigation Program				1

  - Hazard Vulnerability Assessment Project				1

    - Hazard Identification and Prioritization			1

    - Hazard Analysis						2

    - Facility and Resource Identification and assessment		2

  - Methodology								3

    - Theory								3

    - Automation							4

1.4	Format of Analysis							5

# Part II – Antigua

- Antigua - Hurricane and Storms						7

  - Hazard Zones								7

  - Wind									7

    - Wind Zones							7

    - Wind FVS							9

    - Feature Vulnerability to Wind				10

  - Storm Surge								11

    - Storm Surge Zones						11

    - Storm Surge FVS						12

    - Feature Vulnerability to Storm Surge				13

  - Waves									13

    - Wave Zones							13

    - Wave FVS							15

    - Feature Vulnerability to Waves				15

- Antigua – Drought								16

  - Drought Zones							16

  - Drought FVS								16

  - Feature Vulnerability to Drought					17

- Antigua – Flooding								19

  - Flood Zones								19

  - Flood FVS								19

  - Feature Vulnerability to Floods					21

- Antigua – Inland Erosion							21

  - Inland Erosion Zones							21

  - Inland Erosion FVS							21

  - Feature Vulnerability to Inland Erosion				22

- Beach Erosion								23

  - Beach Erosion Zones							23

  - Beach Erosion FVS							23

  - Feature Vulnerability to Beach Erosion				24

# Part III – Barbuda								Page

- Barbuda - Hurricane and Storms						25

  - Wind									25

    - Wind Zones							25

    - Wind FVS							26

    - Feature Vulnerability to Wind				26

  - Storm Surge								27

    - Storm Surge Zones						27

    - Storm Surge FVS						28

    - Feature Vulnerability to Storm Surge				28

  - Waves									29

    - Wave Zones							29

    - Wave FVS							30

    - Feature Vulnerability to Waves				31

  - Barbuda – Drought							31

  - Drought Zones							31

  - Drought FVS								31

  - Feature Vulnerability to Drought					32

- Barbuda – Flooding								32

  - Flood Zones								32

  - Flood FVS								33

  - Feature Vulnerability to Floods					34

- Barbuda – Inland Erosion							34

  - Inland Erosion Zones							34

  - Inland Erosion FVS							35

  - Feature Vulnerability to Inland Erosion				35

- Beach Erosion								35

  - Beach Erosion Zones							35

  - Beach Erosion FVS							35

  - Feature Vulnerability to Beach Erosion				35

Part IV - Summary

12.0	Summary									37

12.1 Cumulative Vulnerability						37

12.2 Antigua									37

12.2.1 Facilities							37

12.2.2 Hazard Zones							38

12.3 Barbuda									39

12.3.1 Facilities							39

12.3.2 Hazard Zones							40

# List of Tables

Table 1 - 	Vulnerability Assessment Factors, Categories and Codes		3

Table 2-	Hazard Priority Scores						4

Table 3-	Highest Possible Facility Vulnerability Score			6

Table 4-	Wind, Waves and Storm Surge Hazard Categories			7

Table 5-	Saffir / Simpson Hurricane Scale					8

Table 6-	Flood Plain Water Depth and Hazard Category			19

List of Maps

Map 1-	 Antigua Wind / Hurricane Vulnerability by Return Period			8

Map 2-	 Antigua Long Term Vulnerability to Hurricane / Wind and Facility FVS	9

Map 3-	 Antigua Facilities with Wind FVS Greater then 21				10

Map 4-	 Antigua Storm Surge Vulnerability by Return Period			11

Map 5-	 Antigua Long Term Vulnerability to Storm Surge and FVS			12

Map 6-	 Antigua Feature Vulnerability to Storm Surge				13

Map 7-	 Antigua Wave Vulnerability by Return Period				14

Map 8-	 Antigua Long Term Vulnerability to Waves					15

Map 9-	 Antigua Drought Zones and Facility FVS					17

Map 10- Antigua Location of Facilities with FVS Greater than 15			17

Map 11- Antigua Location of Existing Agriculture and Grazing			18

Map 12- Antigua Location of Good Agricultural Land				18

Map 13- Antigua Flood Vulnerability Zones and Facility FVS			20

Map 14- Antigua Inland Erosion Zones and Facility FVS				22

Map 15- Antigua Location of Facilities with FVS Greater than 7			22

Map 16- Facilities most Vulnerable to Beach Erosion in Antigua			23

Map 17- Barbuda Wind / Hurricane Vulnerability by Return Period			25

Map 18- Barbuda Long Term Vulnerability						26

Map 19- Barbuda Facilities with Wind FVS Greater then 21			26

Map 20- Barbuda Storm Surge by Return Period					27

Map 21- Antigua Long Term Vulnerability to Storm Surge				28

Map 22- Barbuda Facilities with Highest Storm Surge FVS				29

Map 23- Barbuda Wave Vulnerability by Return Period				30

Map 24- Barbuda Long Term Vulnerability to Waves				30

Map 25- Barbuda Drought Vulnerability Zones and Facility FVS			31

Map 26- Barbuda Flood Vulnerability Zones						32

Map 27- Barbuda Flood FVS								33

Map 28- Barbuda Inland Erosion and Facility FVS					34

Map 29- Barbuda Beach Erosion Zones						36

Map 30- Total “V” of Facilities in Antigua						37

Map 31- Total FVS of Facilities in Antigua						38

Map 32– Facility FVS “Hotspots” in Antigua					38

Map 33- Total Hazard Vulnerability in Antigua					39

Map 34- Distribution of Facility Total FVS in Barbuda				39

Map 35- Total Hazard Vulnerability in Barbuda					40

# List of Figures

Figure 1-	Antigua Shelters with Wind FVS 36					10

Figure 2-	Antigua Facilities with High Storm Surge FVS			12

Figure 3-	Antigua Facilities with High Wave FVS				15

Figure 4-	Antigua Facilities with High Flood FVS				20

Figure 5-	Barbuda Listing of Facilities with High Flood FVS			33

List of Appendices

Appendix 1 – Antigua Facilities by Type and Hazard FVS

Appendix 2 – Barbuda Facilities by Type and Hazard FVS

# PART I – PROJECT AND METHODOLOGY

- Introduction

1.1 Post Georges Disaster Mitigation Program

The Post Georges Disaster Mitigation (PGDM) program is one of three components of The Hurricane Georges Reconstruction and Recovery in the Eastern Caribbean program. The PGDM is responsible for implementing the disaster mitigation capacity building component and seeks to reduce the vulnerability of population and economic activities to natural hazards. Towards this end, the PGDM includes a hazard vulnerability assessment component.

1.2 Hazard Vulnerability Assessment Project

The Hazard Vulnerability Assessment Project consists of an assessment of the hazards to which Antigua and Barbuda are prone and the vulnerability of critical facilities and resources to the hazards. In order to conduct this assessment the project consisted of the following activities:

- Hazard identification and prioritization

- Hazard analysis

- Facility and resource identification and vulnerability assessment

1.2.1 Hazard Identification and Prioritization

Hazard identification and prioritization was conducted during a Hazard Mapping Prioritization Workshop in Antigua during August 2000. The workshop produced a list of six hazards which are listed in order of priority below:

- Winds / Hurricanes

- Drought

- Storm Surge

- Floods

- Coastal and Stream Erosion

- Earthquakes

1.2.2 Hazard Analysis

Specialists in the areas of wind, storm surge, drought, flooding and erosion were contracted to study the vulnerability of Antigua and Barbuda to the hazards and produce detailed reports and hazard vulnerability zone maps. The vulnerability categories used for the maps produced were No vulnerability, low, moderate, high and very high. These reports and maps are available at http://www.oas.org/pgdm.

Hazard vulnerability maps were produced for the following hazards:

- Wind

- Storm surge

- Waves

- Drought

- Flooding

- Inland erosion

- Beach erosion

Wind, storm surge and waves are caused by hurricane and tropical storm activity and are closely related. The Taos model was used to generate the vulnerability maps. Drought was analyzed on the basis of watersheds and considered environmental, meteorological, hydrological, infrastructural, human and land use factors. Flooding considered factors such as slopes, drainage, ratio of watershed area to flood plain and run-off rates and potential. The Hurricane Lenny rainfall event was used as the basis of classification. Inland erosion examined sheet and rill erosion, gullying and landslides. Beach erosion considered only those beaches that are monitored by The Fisheries Department of Antigua and Barbuda.

1.2.3 Facility and Resource Identification and Assessment

The process of facility and resource identification and assessment consisted of the definition of the facilities and resources to be considered, data collection, data automation and finally vulnerability assessment. The facilities and resources to be considered were identified with The Development Control Agency (DCA), The National Office of Disaster Services (NODS) and The Environment Division.

The resources included in the assessment were constrained by the availability of digital mapped resource data. Digital land capability and land use maps provided by the DCA were used as the main source maps. These maps enabled the inclusion of such resources as forestry, tourism and agriculture.

Critical facilities were defined as the following:

- Any facilities that functioned as a shelter

- Hospitals and clinics

- Government administrative buildings

- Airports, Sea ports and Bridges

- Power, Water and Telecommunication Installations

- Oil and Gas Companies

- Protective Services

- Hotels and Guest Houses

- Historical Sites

NODS conducted a survey of all facilities and data on their disaster history, structural and operational vulnerability. The data was collated, coded and entered into a database that was then integrated into a Geographic Information System (GIS). The categories for each of the vulnerability assessment factors and their codes are indicated in Table 1.

###### Table 1 – Vulnerability Assessment Factors, Categories and Codes

| Factor | Category | Code |
| --- | --- | --- |
| Damage History | None | 0 |
|  | Minor | 1 |
|  | Moderate | 2 |
|  | Repetitive / Significant | 3 |
| Structural Vulnerability | No | 0 |
|  | Yes | 1 |
| Operational Vulnerability | No Effect | 0 |
|  | Minimal | 1 |
|  | Significant | 2 |
|  | Life Threatening | 3 |

### 1.3 Methodology

1.3.1 Theory

The vulnerability assessment process was established by Dr. C. Rogers and presented at the Hazard Mapping Prioritization Workshop in Antigua during August 2000. The methodology as set out in the workshop consists of the following steps:

- Identification and prioritization of hazards

- Creation of an inventory of critical facilities

- Assessment of each facility in terms of damage history, structural vulnerability and operational vulnerability for each hazard identified.

- The creation of hazard specific vulnerability zone maps

- The locational assessment of facilities within hazard zones

- The calculation of a total facility vulnerability score (FVS) for each facility and each hazard.

The vulnerability assessment process is defined by the formula :

#### FVS = (L+V)HPS

Where “FVS” is the Facility Vulnerability score, “L” is the Locational Vulnerability, “V”  is the Facility Vulnerability and “HPS” is the Hazard Priority Score.

The “FVS” is a relative score given to each facility to represent its vulnerability to a hazard. The formula incorporates subjective weightings such as the operational vulnerability of the “V” factor and the hazard priority score. The “FVS” should be used as an indication of the need for further examination of facilities. It should be analyzed in terms of the relative scores of locational vulnerability as opposed to facility vulnerability factors such as damage history, structural and operational vulnerability. The analysis of these scores will indicate the most important contributing factor to the vulnerability of the facility and provide a guide to mitigation.

As previously indicated, the workshop generated a priority listing of the six hazards and the hazards were weighted from 1 to 6 to generate a Hazard Priority Score (HPS) as listed below. These are the values utilized in this assessment. The Coastal and Stream Erosion value was applied to Inland Erosion. The Wave hazard generated by storms was assumed to have a value of “1” as it was not considered separately by the workshop.

Table 2 – Hazard Priority Scores

| Hazard | Hazard Priority Score (HPS) |
| --- | --- |
| Winds / Hurricanes | 6 |
| Drought | 5 |
| Storm Surge | 4 |
| Floods | 3 |
| Coastal and Stream Erosion | 2 |
| Earthquakes | 1 |

1.3.2 Automation

The Vulnerability Assessment Project automates the procedure established by Dr. Rogers. It is automated within a GIS environment to allow for the integration and analysis of data with hazard mapping. The system is designed within Microsoft Access TM and ArcView 3.2TM  It consists of a Microsoft Access TM database and an ArcView 3.2TM project for each island.

The Microsoft Access TM database stores the facility data collected by NODS and calculates the “V” of each facility for each hazard. A table consisting of the hazard specific “V” scores is linked to each ArcView 3.2TM   project.  The ArcView 3.2TM project consists of digital maps and linked data tables.

Maps on the following features were integrated into the project in order to identify the features vulnerable to hazards:

- Critical facilities

- Roads

- Settlements

- Corals

- Topography

- Drainage

- Land Use / Land Cover

The GIS facilitates the overlay of hazard maps on the location of features and critical facilities. Visual interpretation determines which features are located in the various hazard zones. Spatial and tabular manipulations in the GIS identify facilities in each hazard zone and calculate the “FVS” of each facility.

  - Format of Analysis

The analysis considers Antigua and Barbuda separately. It considers each hazard and the features and facilities that are most vulnerable to the hazard. The vulnerability of features is indicated by their location in high and very high hazard zones. The vulnerability of facilities is indicated by the value of their “FVS”. Table 3 indicates the highest possible “FVS” for each hazard type. “FVS” are ranked and the facilities with the highest scores are indicated. A FVS was considered high if it was more than 50% of the total possible. A listing of each facility by type, hazard and FVS is attached at Appendix 1.

Table 3- Highest Possible Facility Vulnerability Score

| ### Hazard | Highest Possible FVS |
| --- | --- |
| Wind | 42 |
| Drought | 35 |
| Storm Surge | 28 |
| Floods | 21 |
| Inland and Beach Erosion | 14 |
| Waves | 7 |
| Total FVS | 147 |

In order to identify facilities most vulnerable to all hazards the total FVS was calculated and the facilities noted. The total FVS values were used to create a contour map to easily identify total FVS “hotspots”. A similar approach was used for hazards. The individual hazard maps were converted to grid maps and the scores for all hazard maps were summed to generate a total “vulnerability” map and identify zones most vulnerable to all hazards.

PART II – ANTIGUA

- Antigua - Hurricanes and Storms

2.1 Hazard Zones

Three hazards related to hurricanes and storms were studied: wind, storm surge and waves. Storm surge and waves are dependent upon wind for their generation and the hazards are closely related. Table 4 indicates the categories used to zone these hazards. It indicates the lower and upper bounds of each of the categories and provides a reference for the hazard maps produced. Table 5 describes the Saffir / Simpson Hurricane Scale, it can be used to convert the categories and bounds in Table 4 to measurements and damage estimates.

Table 4 – Wind, Waves and Storm Surge Hazard Categories

Source: Wagenseil, R. 2001. Wind and Storm Surge Technical Report. http://www.oas.org/pgdm.

2.2 Wind

2.2.1 Wind Zones

Map 1 indicates the vulnerability of Antigua to winds by the return periods of 10 years, 25 years, 50 years and 100 years. The 10-year return period subjects the entire island to low vulnerability that is of the tropical storm and hurricane category 2 wind strength. Minimal damage would be expected. The 25-year return period would generate low vulnerability for most of the island with some sections of the southern range experiencing moderate vulnerability. This would create hurricane category 2 winds and moderate damage. For the 50-year return period most of Antigua would be of moderate vulnerability. The western coast would have a low vulnerability with sections of the southern coast subjected to high vulnerability. Category 3 and 4 winds would be expected with extensive and extreme damage. The 100-year storm would place most of the island within the high vulnerability zone. The western third of the island and pockets in the central and eastern districts would have a medium vulnerability. Category 4 winds with extreme damage would be expected.

Table 5- Saffir/ Simpson Hurricane Scale

Map 1- Antigua Wind / Hurricane Vulnerability by Return Period

The long-term vulnerability is similar to the 50-year storm. As Map 2, indicates the central and eastern districts of the island are within the moderate vulnerability zone. The western section of the island is low and some sections of the southern range are within a high vulnerability zone.

2.2.2 Wind FVS

Map 2 indicates the distribution of facilities by wind FVS and the long-term vulnerability to wind. It indicates that facilities with the higher FVS are fairly evenly distributed throughout the north, west and southern sections of the island. Map 3 indicates the location of facilities which have an FVS of more than 50% of the possible wind FVS (42). They are clustered around St. John’s and scattered evenly throughout the southern half of the island.

Map 2- Antigua Long Term Vulnerability to Hurricane / Wind and Facility FVS

The analysis reveals that several key critical facilities have extremely high FVS. The facilities and their FVS are listed below:

Holberton Hospital - 54

Crabbs and Cassada Power Stations - 48

Crabbs Desalination Plat – 48

Friars Hill Power Station - 42

V.C. Bird International Airport – 42

Map 3. Antigua Facilities with Wind FVS Greater than 21

In addition, several Shelters had FVS of 36 and are listed below in Figure 1.

Figure 1. Antigua Shelters with Wind FVS of 36

2.2.3 Feature Vulnerability to Wind

In the long term most of Antigua is moderately vulnerable to winds. The high vulnerability zones consist largely of woodland and grazing lands. The exception is along the southeast coast where the southern section of Dockyard and Shirley Heights are highly vulnerable. Most of St. John’s is vulnerable to low winds.

2.3 Storm Surge

2.3.1 Storm Surge Zones

Map 4 indicates storm surge vulnerability by return period. It indicates that for the 10-year period the entire coast with the exception of the Fitches Creek / Parham Harbour area would experience low storm surge vulnerability. It would be similar to that experienced in a tropical storm with some damage and surge to the heights of 0.1 to 0.5 meters. The Fitches Creek / Parham Harbour area would experience medium storm surge vulnerability with surge varying between 0.5 and 1.5 meters minimal damage.

The 25-year return period would place most of the coast within a moderate vulnerability storm surge zone and the southwestern section of Parham Harbour would be subjected to High vulnerability. Intrusions of moderate storm surge would be expected in the Hanson’s Bay and Jolly Harbour areas. The sea would surge in Parham Harbour to 3.0 meters and cause extensive damage. The 50-year return period increases the area of intrusion around Parham Harbour, Hanson’s Bay and Jolly Harbour. The 100-year return period increases the vulnerability of the Hanson’s Bay area to high and results in high storm surge throughout Parham Harbour.

2.2.1 Storm Surge FVS

2.2.2 Feature Vulnerability to Storm Surge

2.3 Waves

### Map 4- Antigua Storm Surge Vulnerability by Return Period

Map 5. indicates the long term vulnerability of Antigua to storm surge it indicates that Hanson’s Bay and Jolly Harbour are moderately vulnerable to storm surge. It indicates Parham Harbour with a high vulnerability and an inland area to the southwest with moderate and low vulnerability. All bays along the coast would be subjected to moderate storm surge with the exception of those on the southwest that would be low.

Map 5- Antigua Long Term Vulnerability to Storm Surge and FVS

2.3.2 Storm Surge FVS

Eleven facilities have an FVS of more than 14 and Map 5. indicates their distribution. Six of these facilities are hotels on Dickenson Bay / Runaway Bay. Figure 2. lists the facilities. It indicates that the Port of St. John’s, Heritage Quay and adjacent government facilities also have high FVS. Nelson’s Dockyard has an FVS of 16.

Figure 2- Antigua Facilities with High Storm Surge FVS

2.3.3 Feature Vulnerability to Storm Surge

Most of the coastal features of Antigua are vulnerable to moderate and high storm surge. All the Harbours are vulnerable to moderate storm surge. Parham Harbour is vulnerable to high storm surge. In terms of settlement English Harbour and Parham appear to be the only settlements affected. Map 6 indicates that areas of mangrove along the coast will be flooded.

Map 6- Antigua Feature Vulnerability to Storm Surge

2.4 Waves

2.4.1 Wave Zones

Map 7 indicates the vulnerability of Antigua to the various storm return periods. The 10-year return period indicates high vulnerability and very high wave vulnerability on Northern Runaway Beach.  Wave vulnerability is high in the center of Parham Harbour and St. John’s Harbour with a mix of low and moderate vulnerability along the coast. Moderate wave vulnerability intrudes into Hanson’s Bay and Jolly Harbour. Very high vulnerability is experienced at the mouth of English Harbour and decreases inland.

Very high vulnerability zones will experience waves between 2.0 and 10.0 meters. High zones will have waves between 1.5 and 2.0 meters. Moderate vulnerability zones can expect waves between 1.0 and 1.5 and low vulnerability zone will have wanes below 1.0 meters.

The 25-year return period intrudes high waves further into Parham Harbour and low waves inland. This intrusion increases with the 50-year storm and with the 100-year storm some high vulnerability can be expected inland of Parham Harbour. The 100-year storm also results in the intrusion of the high vulnerability zone into St. John’s Harbour, Jolly Harbour and English Harbour.

### Map 7- Antigua Wave Vulnerability by Return Period

Map 8 indicates the long-term vulnerability of Antigua to waves. It indicates the intrusion of a zone of very high vulnerability into Parham Harbour, English Harbour and most of St. John’s Harbour. Jolly Harbour is located in a zone of high vulnerability and there is a zone of low vulnerability to the south and southwest of Parham.

Map 8- Antigua Long Term Vulnerability to Waves

2.4.2 Wave FVS

Seven facilities have high FVS and are listed in Figure 3. They are located on Runaway Bay, Blue Waters Bay, Mango Bay, Dockyard and Coolidge. As expected the locations are closely related to those subjected to storm surge.

Figure 3. Antigua Facilities with High Waves FVS

2.4.3 Feature Vulnerability to Waves

Areas with off shore reefs are somewhat protected but the storm surge does raise the sea level and reduce protection. Very high waves are experienced at sea and within the inner areas of habours. Jolly Harbour and Parham experience very high waves. There is an intrusion of low waves into the area south west of Parham Harbour that is subjected to storm surge.

- Antigua – Drought

3.1 Drought Zones

Drought was analyzed on the basis of watersheds. Antigua was divided into 13 watersheds and the hazard assessed in terms of the “spatial occurrence of vulnerability” factors. The types of factors considered were:

- Environmental and Meteorological

- Hydrological and Infrastructural

- Human and Land Use

Environmental and meteorological factors identified areas with an annual rainfall below 35 inches, exposed to wind and marine influences with shallow soils, slopes greater than 110 and with cactus scrub vegetation.  Hydrological and infrastructural factors identified areas with limited water resources and inadequate dams or ponds for livestock. The human and land use factors considered grazing, crop location and areas with a population density of more than 5,000 persons per square mile.

The northeast and southwest of Antigua are most vulnerable to drought. The eastern and western areas are within a high zone of vulnerability.  Jackson identifies an area at the southeast of Antigua between English Harbour and St. James Club as the watershed most vulnerable to drought.

3.2 Drought FVS

There is a close correlation between high FVS and high vulnerability as indicated in Map 9 below. Most of the facilities with more than 50% of the possible FVS are located within the Very High vulnerability drought zone indicating the strong effect of location on drought FVS. The exceptions are four hotels, Yepton Hotel, Mango Bay Hotel, Half Moon Bay Hotel and Long Bay Hotel. These facilities are located in a high vulnerability zone as indicated on Map 10.

Map 9. Antigua Drought Zones and Facility FVS

Map 10. Antigua Location of Facilities with FVS Greater than 15

3.3 Feature Vulnerability to Drought

The settlements of Coolidge, Piggotts, Potters and Falmouth are located in the “Very High” vulnerability zone. Map 11 Indicates that approximately 50% of the existing agriculture and grazing is located within high and very high vulnerability zones.

Map 11. Antigua Location of Existing Agriculture and Grazing

Map 12. Indicates the location of good agricultural land in relation to drought vulnerability zones. Good agricultural land was defined by DCA on the land capability map as Class 2 and 3 lands, slopes A, B and C and erosion class 01. It suggests that development of agriculture should be encouraged on good agricultural land with low and moderate drought zone.

Map 12. Antigua Location of Good Agricultural Land

4.0 Antigua - Flooding

4.1 Flood Zones

Cooper indicates that the data required for traditional floodplain mapping was not available for Antigua. As a result, the flood analysis “was restricted first to identifying the areas that would generally flood and then to further categorize these areas according to rough estimates of the flood levels expected within the areas”. Areas prone to flooding were identified as those with gentle slopes, poor drainage, large ratios of watershed area to flood plain, rapid run-off to flood plain with high run-off potential.

The hazard category was determined by the water depth of each flood plain on the basis of the Hurricane Lenny rainfall event, using the classification indicated in Table 6. Cooper indicates that this method of categorization provides a measure of the magnitude of flooding to be expected from such an event and provides a measure of the severity of flooding among the identified zones.

Table 6. Flood Plain Water Depth and Hazard Category

| Water Depth (mm) | # Hazard |
| --- | --- |
| >1200 | Very High |
| 1200-600 | High |
| 600-300 | Moderate |
| 300-100 | Low |
| <100 | Very Low |

4.2 Flood FVS

Map 13 indicates the distribution of flood zones and the Facility FVS. The zone of very high vulnerability to flooding is to the north of Bendals Bridge. Most of the island has been categorized as a low vulnerability zone.

Figure 3 lists the facilities with high Flood FVS. Four shelters are included on the list, Villa Primary School, Piggots Seventh Day Adventist Church, Valley High School and New Winthropes Seventh Day Adventist Church. Other facilities with high FVS include Friars Hill and Cassada Power Stations, Crabbs Desalination Plant and Liberta Police Station.

Map 13. Antigua Flood Vulnerability Zones and Facility FVS

Figure 4. Antigua Facilities with High Flood FVS

4.3 Feature Vulnerability to Floods

Flooding occurs in the areas upstream of swamps and mangroves. High flooding vulnerability zones exist around North Sound Bridge and North of Bendals Bridge. The areas affected by flooding are generally agriculture and grazing lands.

5.0 Antigua - Inland Erosion

5.1 Inland Erosion Zones

Lang  indicates that “simple empirical models were used to produce hazard scores for each land unit” and that  “the models integrate the estimated effects of elements of the environment which cause or influence the hazard and produce a score”.  The final score is an estimate of the likely occurrence of the hazard at the location. The scores were classified into 5 equal area classes, very low, low, medium, high and very high groupings. Adding classes and then reclassifying produced the final map.  It combines gullying, mass movement and to some extent stream bank erosion.  Map 14 indicates the inland erosion zones established. The areas with high and very high vulnerability to inland erosion are located within the southwest half of Antigua.

5.2 Inland Erosion FVS

Map 15 indicates that most of the facilities with the highest vulnerability to inland erosion are located within the southwestern half of Antigua. The facility with the highest FVS (12) is the Copper and Lumber Inn located in Dockyard.

Unfortunately, the database assessment of the “V” factor did not differentiate between coastal and inland erosion. As a result, facilities such as Lashings Hotel on Runaway Bay have a high inland erosion FVS. The assumption must be made because of its location on the coast that it is subject to coastal / beach erosion not inland erosion. Therefore, facilities located on the coast should be ignored.

Map 14. Antigua Inland Erosion Zones and Facility FVS

Map 15. Antigua Location of Facilities with FVS Greater than 7

5.3 Feature Vulnerability to Inland Erosion

Mainly woodland and rough grazing occupy the zones with high and very high vulnerability to inland erosion. Some central settlements such as Potter’s, Sea View Farm and Freeman’s are located within these zones. The condition of roads in these areas is adversely affected by erosion.

6.0 Antigua - Beach Erosion

6.1 Beach Erosion Zones

Beach erosion zones are established with the beach setbacks recommended by The DCA and applied to the beaches monitored by The Fisheries Division of Antigua and Barbuda. The Fisheries Division monitors the beach profiles of sixteen beaches in Antigua on a quarterly basis. Emphasis is placed on changes to the beach profile width and data has been collected since 1992. Most of the beaches in Antigua are categorized as medium to high erosion zones. Table 7 indicates the hazard categories and their respective rates of change.

Table 7. Hazard Category and Percentage Change in Beach Profile Width

| Hazard Category | Rate of Change |
| --- | --- |
| Very Low | +3.38 to +5.28 |
| Low | +1.18 to +3.37 |
| Moderate | -0.43 to +1.47 |
| High | -2.34 to –0.44 |
| Very High | -4.25 to –2.35 |

Nine erosion zones have some effect on facilities and features:

- Dickenson Bay

- Runaway Bay North

- Runaway Bay South

- Lignumvitae Bay

- Crab Hill Bay

- Half Moon Bay South

- Dutchman Bay North

- Long Bay

- Fort James Beach

6.2 Beach Erosion FVS

Only two facilities have a FVS of more than 50% of total possible score. These facilities, Lashings and Sunset Cove are both located on Runaway Bay that has a medium rate of erosion. Sunset Cove is the only facility in Antigua located within a beach erosion zone. Map 16 indicates these facilities.

Map 16. Facilities Most Vulnerable to Beach Erosion in Antigua

6.3 Feature Vulnerability to Beach Erosion

The areas affected by the beach erosion zones are generally tourism development areas such as Dickenson Bay / Runaway Bay on the west coast and Long Bay on the east coast. Other features affected are the airport / military zone in the northeast, swamp and mangrove in the southwest and Crab Hill Bay settlement on the south coast. Dickenson Bay, Long Bay and Deep Bay are the areas with high erosion rates that affect facilities and features.

PART III – BARBUDA

7.0 Barbuda - Hurricanes and Storms

7.1 Wind

7.1.1 Wind Zones

Map 17. indicates the expected vulnerability to winds of Barbuda for various return periods. The 10-year and 25-year storms are predicted to place Barbuda in an area of low vulnerability with winds between 17 and 43 meters/second. The 50-year storm will subject most of the island to moderate vulnerability with an area south of Codrington in a low vulnerability zone. Moderate winds are between 43 and 50 meters / second and would be categorized as a category 2 hurricane. The 100-year storm would place most of Barbuda into a zone of high vulnerability to winds which would be of category 3 hurricane force and extensive damage could be expected.

### Map 17. Barbuda Wind Vulnerability By Return Interval

The long-term vulnerability of Barbuda is shown on Map 18. It is similar to the 50-year return storm with a small section on western Palmetto Point experiencing low vulnerability to wind.

Map 18. Barbuda Long Term Wind Vulnerability

7.1.2 Wind FVS

Map 18 indicates the distribution of critical facilities on Barbuda and their wind FVS. They clustered around Codrington and the south coast. Very few facilities have low FVS values. Map 19 selects all the facilities that have an FVS greater than 21. The only exceptions are the Council Hall, Cocoa Point Airport and Hana Thomas Hospital.

7.1.3 Feature Vulnerability to Wind

As Map 18 indicates that most of the features on Barbuda are moderately vulnerable to wind. The area of low vulnerability is to the south of Codrington and is largely occupied by woodlands.

Map 19. Barbuda Features with Wind FVS Greater than 21

7.2 Storm Surge

7.2.1 Storm Surge Zones

Map 20 indicates the storm surge vulnerability of Barbuda for the 10-year, 25-year, 50-year and 100-year storm. It reveals that the 10-year storm is predicted to generate an area of moderate storm surge vulnerability that occupies the lagoon, the mouth of the lagoon and extends into the bird sanctuary. The southern portion of the spit may be breached by the surge that can be expected to range between 0.5 and 1.5 meters.

The 25-year storm subjects the entire northeast of the island to moderate surge vulnerability. The south coast and the peninsula between the lagoon and the south coast are also within an area of moderate vulnerability. The southern portion of the lagoon has a high vulnerability with surge between 1.5 and 3 meters.

The 50-year storm subjects the entrance to the lagoon and its southern portion to high storm surge vulnerability. These areas increase in extent with the 100-year storm to include a section of the south coast and most of the north of the island.

Map 20-. Barbuda Storm Surge by Return Period

Map 21 indicates the expected long-term vulnerability of Barbuda to Storm Surge. The northeast of the island has a low vulnerability. The entire lagoon including the mouth has a high vulnerability with surge building in the south of the lagoon in excess of 3.0 meters. An area of moderate vulnerability extends southwards from the lagoon across the peninsula. The entire coastline has a moderate vulnerability with and intrusion inland along the south coast of low vulnerability.

Map 21. Barbuda Long Term Vulnerability to Storm Surge

7.2.2 Storm Surge FVS

The Facilities with the highest FVS are located along the lagoon in Codrington and on the south coast in the moderately vulnerable area south of the lagoon. Map 22 indicates that the facilities are River Port, Council Hall and The Electricity Power Station with FVS of 24, 16 and 16 respectively.

7.2.3 Feature Vulnerability to Storm Surge

The Bird Sanctuary, lagoon, sea ports and approximately half of the town of Codrington can be expected to be affected by Storm surge in the long term. Most of the inland areas affected by storm surge are occupied by mixed grazing / woodlands.

Map 22 – Barbuda Facilities with Highest Storm Surge FVS

7.3 Waves

7.3.1 Wave Zones

Map 23 indicates the vulnerability of Barbuda to waves for each storm return period. The 10-year storm is predicted to subject the lagoon and north coast to moderate vulnerability that would develop waves of 1 to 1.5 meters. Some intrusions of a low vulnerability zone will occur south and east of the lagoon with waves of 0 to 1 meter. The 25-year storm will increase this intrusion and most the north of the island will be within a low wave hazard zone. The north coast will be within a zone of high wave vulnerability. The 50-year return period will marginally increase vulnerability with the further intrusion of moderate waves inland. The 100-year storm will generate a high wave vulnerability zone in the southern and northern sections of the lagoon.

The long term vulnerability of Barbuda to waves is shown on Map 24. The lagoon is within a zone of high vulnerability. The northern section of the island is generally of low vulnerability and a zone of low vulnerability extends southwards from the lagoon to the central portion of the south coast. The southeast tip of the island has high wave vulnerability.

###### Map 23. Barbuda Wave Vulnerability by Return Period

Map 24. Barbuda Long Term Vulnerability to Waves

7.3.2 Wave FVS

Only one facility has a high wave FVS for Barbuda and that is River Port. It is located on the exposed southern coast in the area of the peninsula prone to storm surge and moderate waves.

7.3.3 Feature Vulnerability to Waves

The areas vulnerable to waves are similar to those vulnerable to storm surge. The lagoon, Bird Sanctuary will be most affected. The eastern sections of Codrington and development along the south coast can expect some damage.

8.0 Barbuda - Drought

8.1 Drought Zones

Barbuda was divided into 10 watersheds that were ranked as low, moderate and high vulnerability zones. Map 25 indicates the drought vulnerability zones for Barbuda. The flats lands surrounding Codrington are considered to be the most vulnerable.

Map 25. Barbuda Drought Vulnerability Zones and Facility FVS

8.2 Drought FVS

The facilities with high drought FVS are all clustered in Codrington with the exception of Cocoa Point Airport. The following facilities have FVS of 20:

- Cocoa Point Airport

- Codrington Airport

- Fire Station

- Water Pumping Station

- Electricity Power Station

- Telephone Sub-Station

- Public Works

8.3 Feature Vulnerability to Drought

Most of the development in Barbuda occurs within the zone of high vulnerability. The town of Codrington and the entire south coast from River Port to Cocoa Point are located in the high zone. Palmetto Point and its agricultural, industrial and tourism development are located in the zone of low vulnerability

9.0 Barbuda - Flooding

9.1 Flood Zones

Map 26 indicates the flood vulnerability zones for Barbuda. A very low vulnerability zone with two small low vulnerability zones to the north and northeast covers most of the island. Codrington is located within a zone of high vulnerability. Two moderate zones of vulnerability are located along the southern section of the island.

Map 26 Barbuda Flood Vulnerability Zones

9.2 Flood FVS

Map 27 indicates the distribution of facility flood FVS. Most of the facilities with high FVS are clustered within Codrington within the zone of high vulnerability to flooding. Four facilities with high FVS are located in the zone of very low vulnerability. Palmetto Point Hotel and Cocoa Point Hotel are located along the south coast. Hana Thomas Hospital and Public Works is located along the edge of the high flood zone in Codrington. Figure 5 lists the facilities and their FVS value.

Map 27 Barbuda Flood FVS

Figure 5 Barbuda Listing of Facilities with High Flood FVS

9.3 Feature Vulnerability to Flooding

The town of Codrington is the most significant feature vulnerable to flooding on the Island of Barbuda. An area of tidal flats and woodland is subjected to low vulnerability in the north. An area of swamp and woodland is moderately to flooding along the south east coast.

10.0 Barbuda - Inland Erosion

10.1 Inland Erosion Zones

Lang indicates that hazards of sheet and rill erosion and of gully erosion in Barbuda are minimal but wind erosion is relatively important and may provide a building development hazard. Rock falls on the edge of and in the highlands are also to be noted. Barbuda shows the highest hazard ratings on areas with parent materials that may have been wind deposited.

Map 28 shows the zones of inland erosion. The southern and western areas of the island are prone to low vulnerability. The eastern section of the island has a moderate vulnerability.

Map 28 Barbuda Inland Erosion and Facility FVS

10.2 Inland Erosion FVS

The facilities on Barbuda are located within the moderate zone of vulnerability. Only River Port has a high FVS value.

10.3 Feature Vulnerability to Inland Erosion

The area of moderate vulnerability is occupied by tidal flats and woodland in the north and grazing / woodland in the east.

11.0 Barbuda - Beach Erosion

11.1 Beach Erosion Zones

Six beaches are monitored quarterly in Barbuda since 1995 by the Fisheries Division. Of the six beaches monitored Two-Foot Bay and Dulcina Bay record accretion as the dominant process. Erosion is more dominant on Palm Beach and Palmetto Point and sections of Cocoa Point vary between accretion and erosion. Beach Erosion in Barbuda varies from very low to very high on Dulcina Bay and Palmetto Point respectively. Map 29 indicates the location of beach erosion zones.

11.2 Beach Erosion FVS

No Facilities in Barbuda were located within Beach Erosion zones. Palmetto Point Hotel and Cocoa Point Hotel were located closet to high and very high Beach Erosion zones.

11.3 Feature Vulnerability to Beach Erosion

The beaches themselves are the features at risk from erosion. Palm Beach places a section of the spit and therefore the lagoon at moderate vulnerability to beach erosion. The beach at Palmetto Point is very vulnerable to beach erosion.

Map 29 Barbuda Beach Erosion Zones and Facility Vulnerability

PART IV - SUMMARY

12.0	Summary

12.1 Cumulative Vulnerability

Cumulative vulnerability attempts to consider the total vulnerability of facilities and areas to hazards in order to determine which facilities and areas are the most vulnerable. The cumulative vulnerability of facilities is the total FVS of all hazard types. The cumulative vulnerability of an area is the total of all the hazard zone scores for the area.

12.2 Antigua

12.2.1 Facilities

The “V” value of facilities considers their damage history, structural and operational vulnerability. An examination of the total “V” value of facilities (Map 30 ) reveals that the facilities with the highest scores are clustered north of St. John’s and in the Runaway Bay / Dickenson bay and Long Bay areas. Map 31. Indicates that the distribution of the total FVS scores is consistent with the distribution of the “V” scores. Map 32. Visually represents the location of the highest FVS values with the use of contours. This method is only for visual impact as it allows “hotspots” to be easily seen. It is not meant to suggest that there is a continuous surface of FVS values.

Map 30. Total “V” of Facilities in Antigua

Map 31. Total FVS of Facilities in Antigua

Map 32. Facility “FVS” Hotspots in Antigua

“Hot spots” are caused by a combination of facility distribution patterns and absolute “FVS” values. Areas with higher facility density have a denser contour pattern. Areas with high density and high “FVS” values appear as dark spots. The location of “hot spots” can be used to prioritize further investigation.

12.2.2 Hazard Zones

Map 33. indicates the total hazard vulnerability zones generated from all hazard vulnerability maps. The values generated are divided into equal classes. The highest total scores are located along the southeast coast in the Shirley Heights area. The area southwest of Parham Harbour also has a high score. The lowest scores are located in a strip across central Antigua.

Map 33. Total Hazard Vulnerability Zones in Antigua

12.3 Barbuda

12.3.1 Facilities

River Port and Codrington Power Station have the two highest total FVS scores. The other facilities with high total FVS are Cocoa Point Hotel, Codrington Fire Station, Codrington Airport and Public Works Map 34 indicates the distribution of these facilities.

Map 34 Distribution of Facility Total FVS in Barbuda

12.3.2 Hazard Zones

The most vulnerable areas in Barbuda are south east of the lagoon and east of the mouth of the lagoon. Map 35 shows these hazard zones.

Map 35  Total Hazard Vulnerability in Barbuda

Appendix 1

# Antigua- Facilities by Type and Hazard FVS

TYPE	NAME	ADDRESS	Shelter	Beach	Surge	Waves	Wind	Inland 	Drought	Flood

Airport	V.C.BIRD INTERNATIONAL AIRPORT	Coolidge	0	0	0	42	2	25	9

Bridges	BENDALS BRIDGE	BENDALS	0	0	0	6	0	5	12

BIG CREEK BRIDGE	VALLEY ROAD	0	0	0	6	2	5	6

FOLLEY GUT BRIDGE	FOLLEY'S	0	0	0	12	2	5	15

GILBERT'S BRIDGE	GILBERTS	0	0	0	12	2	15	15

NORTH SOUND BRIDGE	NORTH SOUND 	0	0	0	12	4	5	21

Churches	ADVENTIST CHURCH	URLINGS	1	0	0	30	0	10	3

ADVENTIST CHURCH	GLANVILLES 	1	0	0	24	0	15	3

ADVENTIST CHURCH	BENDALS	1	0	0	24	2	5	3

ADVENTIST COMMUNITY CENTRE	LIBERTA	1	0	0	18	8	5	3

ANGLICAN CHURCH	PARHAM	1	0	0	18	2	15	3

ANGLICAN SERVANTS QUARTERS	ST. PHILLIPS 	1	0	0	30	2	15	3

BAXTER METHODIST CHURCH	ENGLISH HARBOUR	1	0	0	30	6	20	3

CASSADA GARDENS METHODIST 	CASSADA GARDENS	1	0	0	30	2	20	3

CEDAR GROVE ADVENTIST CHURCH	CEDAR GROVE	1	0	0	30	2	20	3

CEDAR GROVE WESLEYAN HOLINESS 	CEDAR GROVE	1	0	0	30	2	20	9

CHURCH OF CHRIST	BENDALS	1

CHURCH OF GOD PROPHECY	COOKS HILL	1	0	0	24	8	5	3

FIVE ISLAND MORAVIAN CHURCH	FIVE ISLAND	1	0	0	24	8	15	3

FIVE ISLANDS SHILOH CHURCH	FIVE ISLANDS	1	0	0	24	2	15	3

FORT ROAD ADVENTIST CHURCH	FORT ROAD	1	0	0	30	4	20	3

FREEMAN'S VILLAGE METHODIST 	FREEMAN'S VILLAGE	1	0	0	30	6	15	3

GRACE APOSLOTIC CHURCH	LOVELACE ROAD	1	0	0	30	4	20	3

GRAYS HILL PENTECOSTAL	PERRY BAY	1	0	0	24	0	20	3

GREEN BAY NAZARENE CHURCH	GREEN BAY	1	0	0	24	2	20	3

JEVHOVAH WITNESS KINGDOM HALL	GAMBLES TERRACE	1	0	0	24	4	20	3

JOHN HUGHES METHODIST CHURCH	JOHN HUGHES	1	0	0	36	8	5	3

LOVELACE ADVENTIST CHURCH	LOVELACE ROAD	1	0	0	24	0	20	3

METHODIST CHAPEL	BETHESDA	1	0	0	36	2	15	3

METHODIST CHAPEL AND HALL	PARHAM	1	0	0	36	2	15	3

METHODIST HALL	FREETOWN	1	0	0	24	2	15	3

Wednesday, May 09, 2001	Page 1 of 7

TYPE	NAME	ADDRESS	Shelter	Beach	Surge	Waves	Wind	Inland 	Drought	Flood

Churches	MOUNT OF BLESSING CHURCH	WIRELESS ROAD	1	0	0	30	2	20	3

NAZARENE CHURCH	VILLA	1	0	0	36	0	20	3

OLD ROAD ANGLICAN CHURCH	OLD ROAD	1	0	0	36	0	10	3

PERRY BAY PEOPLE CHURCH	PERRY BAY	1	0	0	24	2	20	3

PILGRIM HOLINESS CHURCH	NEW WINTHROPES	1	0	0	24	2	20	3

SEVENTH DAY ADVENTIST	NEW WINTHROPES	1	0	0	30	2	20	12

SEVENTH DAY ADVENTIST	PIGOTTS	1	0	0	24	0	20	12

SHILO GOSPEL HALL	ST. JOHNSON 	1	0	0	30	2	20	3

ST. JOHN'S CHURCH OF CHRIST	GOLDEN GROVE	1	0	0	30	4	5	3

ST. JOSEPH ANGLICAN CHURCH	URLINGS	1	0	0	30	0	10	3

ST.MARKS	PIGOTTS	1	0	0	30	4	20	3

TINDALE ADVENTIST CHURCH	VALLEY ROAD	1	0	0	30	8	5	3

VILLA BETHEL CHURCH	VILLA	1	0	0	30	0	20	3

VILLA ST. ANDREW'S CHURCH	VILLA	1	0	0	30	4	20	3

WESLEY HOLINESS	BOLANS	1	0	0	24	6	10	3

ZION CHURCH OF GOD	VILLA	1	0	0	30	0	20	3

Clinics	ALL SAINTS HEALTH CENTRE	A l Saints	0	0	0	30	8	5	3

BENDALS CLINIC	Bendals Rd.	0	0	0	30	2	5	3

BETHESDA CLINIC	BETHESDA	0	0	0	12	2	15	3

BISHOPGATE STREET CLINIC	ST. JOHN'S	0	0	0	12	0	20	3

BOLANS CLINIC	Bolans Village	0	0	0	24	8	10	3

CEDAR GROVE CLINIC & CRES	Cedar Grove	0	0	0	30	2	20	3

CLARE HALL & ST. JOHNSON'S VILLAGE	Clare Hall Village	0	0	0	18	4	20	3

COBBS CROSS CLINIC	Cobbs Cross Villag.	0	0	0	30	2	20	3

FREETOWN CLINIC	Freetown	0	0	0	30	2	15	3

GRAYS FARM HEALTH CENTRE	Grays Farm	0	0	0	30	2	20	3

JENNINGS CLINIC	Jennings	0	0	0	18	2	10	3

JOHN HUGHES CLINIC	John Hughes	0	0	0	36	0	5	3

JUDGES HILL CLINIC	Judges Hill	0	0	0	30	2	20	3

LIBERTA CLINIC	Liberta	0	0	0	30	2	5	12

NEWFIELD CLINIC	NEWFIELD	0	0	0	36	2	5	3

OLD ROAD CLINIC	Old Road	0	0	0	36	0	10	3

OTTOS NEWTOWN CLINIC	Ottos New Town	0	0	0	6	4	20	3

PARES CLINIC	Pares Village	0	0	0	36	2	15	3

PARHAM CLINIC	Parham Village	0	0	0	36	2	15	3

Wednesday, May 09, 2001	Page 2 of 7

TYPE	NAME	ADDRESS	Shelter	Beach	Surge	Waves	Wind	Inland 	Drought	Flood

Clinics	PIGOTTS CLINIC	0	0	0	12	4	20	3

POTTERS CLINIC & COMMUNITY 	Potters main Road	0	0	0	30	8	5	3

ST. JOHN'S HEALTH CENTRE	BROWN'S AVENUE	0	0	0	6	0	20	3

ST. JOHN'S HEALTH CENTRE	All Saints  d.	0	0	0	36	0	10	3

SWEETS CLINIC	Sweets Village	0	0	0	36	4	5	3

WILLIKIES CLINIC	Wilikies	0	0	0	30	2	15	3

Community 	ADVENTIST CHURCH	BOLANS	1	0	0	24	2	10	3

CATHEDRAL CULTURAL CENTRE	ST. JOHN'S STREET	1	0	0	30	0	20	12

COMMUNITY CENTRE	YORKS	1	0	0	24	4	20	3

Electricity	CASSADA POWER STATION	BARNES HILL MAIN 	0	0	0	48	2	20	12

CRABS POWER STATION	CRABBS	0	0	0	48	0	15	9

FRIARS HILL POWER STATION	FRIARS HILL ROAD	0	0	0	42	4	20	18

Fire	ALL SAINTS FIRE STATION	All Saints	0	0	0	36	8	5	3

ST. JOHNS FIRE STATION	Factory Rd.	0	0	0	42	0	20	9

V.C. BIRD FIRE STATION	Airpo t, Fire Station	0	0	0	30	2	20	3

Government 	AGRICULTURE EXTENTION OFFICES	Valley Rd.	0	0	0	30	0	20	3

ALLIANCE FOR SOCIAL WELL BEING	0	0	0	36	0	20	3

ANTIGUA & BARBUDA TRANSPORT 	Factory Road	0	0	0	24	4	5	3

APUA MAIN ADMINISTRATION OFFICE	CASSADA GARDENS	0	0	0	24	2	20	3

BOARD OF EDUCATION	North Street	0	0	0	24	0	20	3

CENTRAL HOUSING & PLANNING	All Saints Rd.	0	0	0	24	0	20	3

CLARENCE HOUSE	English Harbour	0	0	0	36	0	20	3

CUSTOMS & EXCISE DEPARTMENT	Church St.	0	0	0	30	0	20	3

DEPARTMENT OF COOPERATIVES	Upper High Street	0	0	0	24	0	20	3

DEPARTMENT OF TOURISM	Nevis Street	0	16	3	24	0	20	3

ESTABLISHMENT	Redcliffe St.	0	0	0	18	0	20	3

FREE TRADE ZONE	COOLIDGE	0	0	0	24	2	20	3

GENDER AFFAIRS (Craft)	Cross Street	0	2	0	1	24	2	20	9

GENERAL POST OFFICE	High & Long St.	0	12	0	36	0	20	12

GOVERNMENT OFFICE COMPLEX	Queen Eliz. Hgh.	0	4	0	2	18	8	20	3

GOVERNMENT PRINTERY	Factory Rd.	0	0	0	42	4	20	9

GOVERNMENT WORKSHOP 	Factory Road	0	0	0	36	4	20	3

GOVERNMENT WORKSHOP OFFICE	Factory Road	0	0	0	30	8	20	12

H.C. GRANT BUILDING	ST. JOHN'S	0	8	0	24	0	35	12

HEALTH EDUCATION UNIT	Factory Rd.	0	0	0	36	6	20	3

Wednesday, May 09, 2001	Page 3 of 7

TYPE	NAME	ADDRESS	Shelter	Beach	Surge	Waves	Wind	Inland 	Drought	Flood

Government 	HEALTH INFORMATION DIV. / AIDS 	Cemetry Road	0	0	0	24	4	20	3

HIGH COURT	0	0	0	42	6	20	9

HUMAN RESOURCES CENTRE	Factory Rd.	0	0	0	30	6	20	3

IMMIGRATION DEPARTMENT	Old Parham Rd.	0	0	0	24	4	20	3

INDUSTRIAL COURT	Redcliffe St.	0	0	30	0	20	3

INLAND  REVENUE	Newgate St.	0	0	0	24	0	20	3

JOB PROGRAMME	Factory Rd.	0	0	0	30	4	20	9

MAGISTRATE COURT	Nevis Street	0	0	0	24	0	20	3

MIN. AGRIC., LAND & FISHERIES	Nevis Street	0	0	0	30	0	20	3

MIN. COMMERCE, INDUSTRY, 	Corn Alley	0	0	0	24	0	20	3

MIN. OF JUSTICE & LEGAL AFFAIRS	Redcliffe Street	0	0	0	24	0	20	3

MIN. OF LABOUR (HEADQUARTERS)	Redcliffe St. & Corn	0	0	0	18	0	20	3

MIN. OF TOURISM & ENVIRONMENT / 	Queen Eliz. Hgh.	0	0	0	30	4	20	9

MINISTRY OF AGRICULTURE 	0	0	0	42	8	5	12

MINISTRY OF EDUCATION	Church St.	0	0	0	48	0	20	9

MINISTRY OF PLANNING	Church St.	0	0	0	18	0	20	3

MINISTRY OF PUBLIC WORKS	ST. JOHN'S STREET	0	0	0	36	0	20	3

NAT. DRUG MONEY LAUNDERING CO.	0	0	0	24	0	20	3

NATIONAL OFFICE OF DIASTER 	American Rd.	0	0	0	36	4	5	3

OLD ADMIN. MIN. OF FINANCE	High & Long St.	0	16	0	30	0	20	12

OMBUDSMAN OFFICE	Deanery Lane	0	0	0	30	0	20	3

PARLIAMENT BUILDING	Queen Eliz. Hgh.	0	2	0	1	30	6	20	3

PLANT PROTECTION UNIT	Friars' Hill	0	0	0	12	2	20	3

PRIME MINISTER OFFICE	Queen Eliz. Hgh.	0	0	0	12	4	20	3

TREASURY	High Street	0	0	0	30	0	20	3

Guest Houses	PARADISE INN	0	0	0	24	4	25	6

STEPHENDALE	0	0	0	36	4	25	6

VIENNA INN	0	0	0	30	4	25	3

Historical 	BETTY'S HOPE	BETTY'S HOPE 	0	0	0	24	4	15	3

FORT BARRINGTON	GOAT HILL	0	0	0	18	0	3

FORT JAMES	FORT JAMES, ST. 	0	0	0	42	0	20	3

MONKS HILL	FALMOUTH	0	0	0	6	8	5	3

NELSON'S DOCKYARD	DOCKYARD, 	0	6	16	6	30	12	3

SHIRLEY HEIGHTS	ENGLISH H RBOUR	0	0	0	36	0	20	3

Hospitals	HOLBERTON HOSPITAL	Queen Eliz. Hgh.	0	0	0	54	6	35	9

Wednesday, May 09, 2001	Page 4 of 7

TYPE	NAME	ADDRESS	Shelter	Beach	Surge	Waves	Wind	Inland 	Drought	Flood

Hospitals	MENTAL HOSPITAL	Skerrits Pasture	0	0	0	36	4	20	3

MT. ST. JOHN'S MEDICAL CENTRE	MIHAEL'S MOUNT	0	0	0	6	0	20	3

Hotels	ADMIRALS INN	DOCKYARD	0	0	0	6	6	20	3

AIRPORT HO EL	0	0	0	36	2	25	3

AMARYLIS HOTEL	AIRPORT ROAD	0	0	0	36	2	20	18

BARRYMORE BEACH HOTEL	RUNAWAY BAY	0	20	4	36	0	20	12

BEACHOMBER HOTEL	COOLIDGE	0	8	4	12	2	20	3

BLUE WATERS HOTEL	BLUE WATERS BAY	0	8	4	12	2	20	3

CARLISLE BAY HOT L	0	2	4	1	36	2	15	6

CATAMARAN	FALMOUTH	0	0	0	12	0	20	3

CITY VIEW HOTEL	0	0	0	18	0	20	3

COPPER AND LUMBER INN	DOCKYARD	0	4	3	6	6	20	3

COURTSLAND HOTEL	0	0	0	36	2	25	3

CURTAIN BLUFF	OLD ROAD	0	0	0	12	0	10	3

DOVE COVE HOTEL	DRYHILL ESTATE	0	0	0	12	0	20	3

FALMOUTH HARBOUR BEACH 	FALMOUTH	0	0	0	12	0	20	3

FALMOUTH HARBOUR HOT L	0	2	8	1	30	8	25	3

FALMOUTH HOTEL	0	2	8	1	30	8	25	6

GALLEY BAR AND BOUTIQUE	DOCKYARD	0	0	12	0	20	3

GALLEY BAY HOTEL	FIVE ISLANDS	0	0	0	12	0	15	3

GULF VIEW HOTEL	NEW GATE STREET	0	0	0	12	0	20	3

HALE HOUSE	0	2	12	1	24	2	20	6

HALF MOON BAY HOTEL	0	6	12	3	30	6	20	6

HAWKSBILL BEACH RESORT	FIVE ISLANDS	0	0	0	6	0	15	3

HERITAGE HOTEL	HERITAGE QUAY, 	0	6	12	3	24	6	20	12

ISLAND INN	ANCHORAGE ROAD	0	0	0	24	4	20	3

JOE MIKES HOTEL	NEVIS STREET AND 	0	0	0	24	0	20	3

JOLLY CASTLE	BOLANS	0	0	0	6	0	10	3

JOLLY HARBOUR	BOLANS	0	0	0	6	0	10	3

JUMBY BA  RESORT	0	8	0	36	0

LASHINGS	RUNAWAY BAY	0	10	28	9	36	10	18

LONG BAY RESORT	0	2	12	30	4	20	6

LORD N LSON CLUB	0	4	0	30	2	25	6

MANGO BAY RESORT	0	6	16	7	30	6	20	6

MARINA BAY HOTEL	0	2	12	1	24	2	25	9

Wednesday, May 09, 2001	Page 5 of 7

TYPE	NAME	ADDRESS	Shelter	Beach	Surge	Waves	Wind	Inland 	Drought	Flood

Hotels	MILL REEF HOTEL	0	4	0	2	42	4	15	3

OCEAN VIEW	0	4	8	2	30	6	25	12

PELICAN ISLE H TEL	0	0	0	30	0	15	3

PINEAPPLE RESORT	0	2	8	1	36	2	6

REX BLUE HERON	0	0	0	12	0	3

REX HALYCON COVE	DICKENSON BAY	0	0	0	6	0	20	3

ROYAL ANTIGUAN	0	0	0	12	6	15	3

SANDALS	DICKENSON BAY	0	6	16	3	30	6	9

SANDPIPER HOTEL	0	4	12	2	30	6	25	9

SIBONEY	DICKENSON BAY	0	6	20	3	36	6	20	15

ST. JAMES CLUB	0	4	12	2	36	4	20	3

ST. JAMES CLUB VILLA	MAMORA BAY	0	0	0	12	0	20	3

SUNSAIL CLUB COLONNA	HODGES BAY	0	4	1	12	0	20	3

SUNSET COVE HOTEL	RUNAWAY  BAY	0	8	20	6	36	4	20	15

TRADEWINDS HOTEL	DICKENSON BAY	0	0	0	30	0	20	3

YEPTON HOTEL	0	2	4	1	24	2	20	3

Military	ANTIGUA & BARBUDA DEFENCE 	Crabbs ( Parham)	0	0	0	12	0	15	3

Nursery 	GRACE HILL PRE-SCHOOL	LIBERTA	1	0	0	24	2	5	6

Other 	DELUXE THEATRE	HIGH STREET	1	0	0	30	0	20	9

KING GEORGE THE V GROUND	KING GEORGE PARK	1	0	0	24	0	20	3

Petroleum	TEXACO WEST INDIES	COOLIDGE	0	0	0	36	2	20	9

WEST INDIES OIL REFINERY	FRIAR'S HILL ROAD	0	0	0	24	4	20	9

Police	ALL SAINTS POLICE STATION	All Saints Village	0	0	0	36	6	5	3

BOLANS POLICE STATION	Bolans	0	0	0	30	2	10	3

COLLIDGE POLICE STATION	Collidge	0	0	0	30	2	20	9

DOCKYARD POLICE STATION	English Harbour	0	0	0	30	6	20	9

FREETOWN POLICE STATION	Freetown	0	0	0	30	6	15	3

GRAYS FARM POLICE STATION	Grays Farm	0	0	0	30	2	20	3

LIBERTA POLICE STATION	Liberta Village	0	0	0	24	2	5	12

PARHAM POLICE STATION	Parham town	0	0	0	36	0	15	3

POLICE HEADQUATERS	American Rd.	0	0	0	36	4	25	3

ST. JOHNS POLICE STATION	Newgate St.	0	0	0	30	0	20	3

WILIKIES POLICE STATION	Willikies Village	0	0	0	30	2	15	3

Primary 	BENDALS PRIMARY SCHOOL	Bendals Village	1	0	0	24	6	5	3

BETHESDA PRIMARY SCHOOL	Bethesda	1	0	0	30	6	15	3

Wednesday, May 09, 2001	Page 6 of 7

TYPE	NAME	ADDRESS	Shelter	Beach	Surge	Waves	Wind	Inland 	Drought	Flood

Primary 	COBBS CROSS PRIMARY SCHOOL	Cobbs Cross	1	0	0	30	2	20	3

FIVE ISLANDS PRIMARY SCHOOL	Five Islands	1	0	0	24	6	15	3

FREEMAN'S VILLAGE PRIMARY	FREEMAN'S VILLAGE	1	0	0	36	6	15	3

GLANVILLES PRIMARY SCHOOL	Glanvilles	1	0	0	30	2	15	3

GREEN BAY PRIMARY SCHOOL	Green Bay	1	0	0	24	4	20	3

IRENE WILLIAMS PRIMARY SCHOOL	Sweets Village	1	0	0	12	2	5	3

JENNINGS PRIMARY SCHOOL	Jennings	1	0	0	24	0	10	3

JOHN HUGHES PRIMARY SCHOOL	John Hughes	1	0	0	30	2	5	3

LIBERTA PRIMARY SCHOOL	Liberta	1	0	0	18	8	5	3

MARY E. PIGOTT PRIMARY SCHOOL	Ottos	1	0	0	24	6	20	3

OLD ROAD PRIMARY SCHOOL	Old Road	1	8	0	36	0	10	3

PARES PRIMARY SCHOOL	Pares Village	1	0	0	24	2	15	9

PARHAM PRIMARY SCHOOL	Parham Village	1	0	0	36	2	15	9

T. N. KIRNON SCHOOL	Coronation Rd.	1	0	0	36	4	20	9

URLINGS PRIMARY SCHOOL	Urlings	1	0	0	36	0	10	3

VILLA PRIMARY SCHOOL	1	0	0	30	0	20	12

WILIKIES PRIMARY SCHOOL	Wilikies	1	0	0	36	2	15	3

Prisons	PRISON	Coronation Rd.	0	0	0	12	0	20	3

PRISON ( REHABILIATION CENTRE)	Factory Rd.	0	0	0	42	6	20	3

Sea Port	DEEP WATER HARBOUR	ST. JOHN'S	0	4	20	2	12	4	20	3

ENGLISH HARBOUR	DOCKYARD, 	0	2	8	1	6	2	6

HERITAGE QUAY	HERITAGE QUAY, 	0	4	16	2	6	4	20	3

JOLLY HARBOUR	JOLLY BEACH	0	0	0	6	0	10	6

Secondary 	ALL SAINTS SECONDARY SCHOOL	All Saints Village	1	0	0	30	4	5	3

ANTIGUA GIRL HIGH SCHOOL	NEWGATE STREET, 	1	0	0	36	0	20	9

ANTIGUA GRAMMER SCHOOL	Old Factory Road	1	0	0	36	4	20	3

CLARE HALL SECONDARY SCHOOL	Clare hall	1	0	0	30	4	20	3

VALLEY HIGH	BOLANS	1	0	0	24	0	10	12

Telecommunic	BOGGY PEAK	BOGGY PEAK	0	0	0	30	8	10	3

PUBLIC INFORMATION & A.B.S RADIO	OLD PARHAM 	0	0	0	24	4	20	6

Water	CRABS DESALINATION PLANT	CRABBS PENINSULA	0	4	0	48	2	15	18

DELAPS PUMPING AND TREATMENT 	BODKIN'S ESTATE, 	0	0	0	36	2	5	3

GRAYS HILL RESERVOIR AND PUMPING	GRAYS HILL	0	0	0	30	8	5	3

Wednesday, May 09, 2001	Page 7 of 7

Appendix 2

Barbuda Facilities by Type and Hazard FVS

Type	Name	Address	Shelter	Wind	Waves	Surge	Drought	Flood	Inland

Airport

COCO POINT AIRPORT	COCO POINT	0	12	1	12	20	12	2

CODRINGTON AIRPORT	CODRINGTON	0	30	0	8	20	15	2

Churches

LIVING FAITH CHURCH	CODRINGTON	1	30	0	0	15	18	0

PENTECOSTAL ASSEMBLY	CODRINGTON	1	30	0	0	15	18	2

PEOPLE CHURCH	CODRINGTON	1	30	0	0	15	15	0

WESLYN HOLINESS CHURCH	CODRINGTON	1	30	0	0	15	15	0

Electricity

ELECTRICITY POWER STATION	0	42	1	16	20	9	0

Fire

BARBUDA FIRE STATION	0	30	1	8	20	18	2

Government Administration

BARBUDA COUNCIL HALL	CODRINGTON	0	12	1	16	15	9	0

NEW BARBUDA COUNCIL 	CODRINGTON	0	24	0	4	15	9	0

POST OFFICE AND TREASURY	CODRINGTON	0	30	0	12	15	9	0

PUBLIC WORKS	0	36	0	0	20	15	2

Historical

MARTELO TOWER	0	24	1	12	5	12	2

Hospitals

HANA THOMAS HOSPITAL	CODRINGTON	0	18	1	4	15	12	2

Hotels

CAY CLUB	0	30	0	12	15	9	0

COCO POINT HOTEL	COCO POINT	0	30	2	12	15	9	4

Wednesday, May 09, 2001	Page 1 of 2

Type	Name	Address	Shelter	Wind	Waves	Surge	Drought	Flood	Inland

PALMETTO POINT HOTEL	PALMETTO POINT	0	30	1	8	15	12	2

Police

BARBUDA POLICE STATION	CODRINGTON	0	30	0	4	15	18	0

Sea Port

CODRINGTON WHARF	0	24	2	12	15	3	2

RIVER PORT	0	36	4	24	5	6	8

Telecommunication

TELEPHONE SUB-STATION	0	30	0	4	20	15	0

Water

WATER PUMPING STATION	0	30	0	0	20	9	2

Wednesday, May 09, 2001	Page 2 of 2

Rogers, C. 2000, Hazard Mapping / Vulnerability Assessment Prioritization Workshop Report.  http://www.oas.org/pgdm

“V” is defined as the total of the damage history, structural vulnerability and operational vulnerability scores.

Jackson, I, 2001, Drought Technical Report – Antigua and Barbuda. http://www.oas.org/pgdm

Cooper, V. 2001, Flood Methodology Notes. Email .

Lang, D.M., 2001, ‘Inland’ Erosion Hazards in Antigua, Barbuda.  http://www.oas.org/pgdm

James, P. 2001, Beach Erosion Technical Report – Antigua and Barbuda. http://www.oas.org/pgdm