# POST-GEORGES DISASTER MITIGATION PROJECT, ST. KITTS AND NEVIS

## CRITICAL FACILITIES ANALYSIS

CALCULATION OF FACILITY VULNERABILITY SCORES (FVS)

### INSTRUCTIONS

- Print the following Excel files: St.  Kitts FVS template,  Nevis FVS template

- Each template (file) has 5 sheets; one sheet for each of the 5 types of facilities included:

Sheet 1 – shelters

Sheet 2 – schools

Sheet 3 - health facilities

Sheet 4 – government buildings

Sheet 5 – utilities.

You may create more templates for other types of facilities by changing the names on the template

- Each sheet has 6 pages, five of which are to be used to calculate the FVS values * for the relevant facility type for each hazard*, and the last page (page6) is to be used to enter the FVS values for that critical facility type, e.g. for all schools

*For St. Kitts - wind, storm surge, coastal erosion, inland erosion, flood

*For Nevis - wind, storm surge, coastal erosion, drought, flood

- Note the following symbols used on the individual sheets:

| SK | ST. KITTS | SH | SHELTER |
| --- | --- | --- | --- |
| N | NEVIS | SCH | SCHOOL |
|  |  | HF | HEALTH FACILITY |
| W | WIND | GB | GOVT BUILDING |
| SS | STORM SURGE | UT | UTILITY |
| CE | COASTAL EROSION |  |  |
| IE | INLAND EROSION |  |  |
| F | FLOOD |  |  |
| D | DROUGHT |  |  |

Thus the SK_SH_W  sheet should be used to determine the FVS values for shelters to the wind hazard in St. Kitts

2.	Use the following equation to calculate the FVS values

FVS  = (L + V) HPS

L  - Locational risk, V – Vulnerability, HPS – Hazard Priority Score

V  =  DH + S + O where:

DH – Damage history

S –     Structural vulnerability

O –    Operational vulnerability

SEE BELOW FOR ASSOCIATED VALUES

Thus, using the SK_SH_W sheet as an example, do the following:

- Enter the names of the shelters

- Enter a code for each shelter, if you wish e.g. sh1, sh2 etc.

- Determine the HPS (Hazard Priority Score) values for each hazard by ranking the hazards from 1 to 5 (1 lowest, 5 highest) according to their relative importance.

- For St. Kitts, rank wind, storm surge, coastal erosion, inland erosion, flood

- For Nevis, rank wind, storm surge, coastal erosion, drought, flood

- Determine the L value  (0 to 4)

- Superimpose the map of the relevant critical facility on the relevant hazard map and then determine the L value using the table below.  For e.g. if we are determining the L value for shelters to the wind hazard, then superimpose the map of shelters on the wind hazard map and determine the L value using Table 1 below.

Table 1

| Hazard category | Locational risk | Meaning |
| --- | --- | --- |
| Very low | 0 | No risk |
| Low | 1 | Minimal risk |
| Moderate/medium | 2 | Possible/probable |
| High | 3 | Probable /high |
| Very high | 4 | Extreme |

- Determine DH, S and O (0 to 3), based on existing data or expert opinion using Tables 2 to 4.

| ## Table 2: Damage History (DH) |  |
| --- | --- |
| 0 | None |
| 1 | Minor |
| 2 | Moderate |
| 3 | Repetitive/signficant |
| ## Table 3: Structural Vulnerability (S) |  |
| 0 | Exceeds code |
| 1 | Meets code |
| 2 | Don’t meet code |
| 3 | Known deficiencies |
| ## Table 4: Operational Vulnerability (O) |  |
| 0 | No effect |
| 1 | Minimal |
| 2 | Significant |
| 3 | Life threatening |

- Determine V as DH + S + O.

- Calculate FVS = (L + V) HPS, and enter on chart.

- Enter FVS values calculated for this shelter on page 6. See below for use of page 6.

- Repeat steps 3 to 8 for schools, health facilities and other types of critical facilities you selected.

- Fill in Nevis FVS template using the same procedure described above.

#### Use of page 6 – ENTRY OF FVS VALUES

- Enter the names of the shelters

- Enter the FVS values for each shelter associated with each hazard.

HAPPY FVS- ING!

THE END!