Inland Erosion Hazard Assessment and Mapping

for Antigua, Barbuda and St. Kitts:

Appendices

Post-Georges Disaster Mitigation Project

in Antigua & Barbuda and St. Kitts & Nevis

April 2001

This report was prepared under contract with the OAS by David Lang, Dominica

Soils

This section deals with the stability characteristics of the soils. It is convenient to consider them in relation to their geomorphological situation rather than pedological classification so that the relationship with the geology and geomorphology is clear. The discussion in this section is based on the work of Hill in Antigua, Lang in Barbuda and Carroll in St. Kitts.

Each island will be dealt with separately, but a description of the broad groupings will assist in understanding the differences between the islands. All three islands lie on a volcanic base, but at the present Barbuda and parts of Antigua are occupied by sedimentary materials which succeeded the volcanic developments of the older volcanic arc. St. Kitts, on the other hand, is almost entirely volcanic, and is wholly within the younger volcanic arc. It consists principally of very youthful volcanics, with the exception of the southeast peninsula where older volcanoes, still a great deal younger than those present in Antigua are found

Antigua

Soils of Antigua can be considered in five geomorphic groups:

- soils of the alluvial areas

- soils of the undulating limestone region

- soils of the moderate slopes of the central region

- soils of the hills of the central region

- soils of the southwest mountains

Soils of the alluvial areas – soils over deep alluvium on gentle slopes

These can be divided into two main groups.

1. Moderately light soils with good drainage. These are deep, with rapid through-drainage, and very low hazard of erosion, which are unlikely to be candidates for mass movement.

Variants which may be erodible are types with occasional compacted subsoils and saline profiles.

2. Heavy soils with imperfect drainage. These occupy very low-lying areas with mainly rather slow drainage. The heavy clays of which they are composed are resistant to raindrop and soil wash erosion – the hazard is very slight. They overlie deep alluvium

Neither group overlies materials likely to participate in mass movement and of course are in a topographic position where such movement is highly unlikely

Soils of the limestone region– soils over massive calcareous materials

These can also be divided into two groups.

1. Light- textured, very shallow soils over calcareous sandstones. These soils are only six to twelve inches deep and overlie materials which are of low permeability so that erosion hazard is moderate or worse. The sites occupied by these soils are candidates for rock falls rather than debris slides

2. Heavy-textured, shallow, well-drained soils over limestone and calcareous marls or grits. They are mostly quite shallow with moderate to slight erosion hazard, because of their structure characteristics and overlie materials which are of low permeability except through fissures– leading to rock fall potential

Soils of the gentle slopes of the central region

These can be divided into three main groups.

1. Moderately heavy, textured, shallow soils with imperfect drainage over gravels. These are of variable drainage and low erosion hazard

2. Heavy textured, moderately deep or deep soils with imperfect or poor drainage over clays. These soils have slow drainage but only have a slight erosion hazard because of the heavy (cohesive) and calcareous (strongly structured) nature of the clay.

3. Heavy, moderately deep or deep soils, with imperfect or poor drainage over clay. These have generally slight to moderate erosion hazard.

Soils of the hills of the central region

1. Shallow, well-drained, light textured soils. These are highly erodible soils.

2. Shallow, moderately heavy textured, well drained soils. These soils are reported to have a moderate to severe erosion hazard despite their heavy texture and free drainage.

3. Heavy-textured, shallow, well drained soils over tuff. These have a severe erosion hazard despite free drainage.

Soils of the southwest mountains

Again, these can be divided into two groups.

1. Soils over igneous rocks. These comprise light-textured, shallow, well drained soils over dacite and moderately heavy textured soils over andesites and basalts. The erosion hazard ranges from moderate to very severe.

2. Moderately heavy-textured, shallow, well drained soils over tuffs and tuffaceous agglomerate. The erosion hazard is moderate to severe.

Barbuda

Only four soil types and one land type need to be considered. The two soils on the limestone highland are both of low erosion hazard with respect to water erosion and low to moderate erosion hazard from wind. The upland between the highland and the beach is occupied by a terrace about fifteen feet above present sea level. Two soils occur on this terrace:

- a silty, heavy-textured soil derived from weathered limestone which is of low erosion hazard by both wind and water, and

- a lighter-textured soil derived from oolitic limestone rubble which is extremely susceptible to wind erosion and has a moderate susceptibility to water erosion.

The extensive area of beach sand is subject to very high wind erosion hazard and little or no erosion hazard from land-based water.

St. Kitts

Soils of St. Kitts can be divided into two main groups on geomorphological grounds.

- those of the southeast peninsula, where older residual hills are connected by recent beach and other marine deposits

- those of the younger volcanos, piles and glaces, of the northwest.

The southeast group subdivides naturally into

1. Soils of the older residual hills, moderately heavy to heavy, in some cases mainly colluvial. These are somewhat unstable soils, liable to creep and erode.

2. Soils of the low lands between the residual hills of the peninsula developed mainly from beach sand and colluvium. These are low lying, variably drained, sometimes saline and with low erosion hazard.

3. Soils of the lower slopes of the residual hills - mainly light textured, colluvial soils - freely drained, somewhat unstable and erodible.

The northeast group comprises roughly concentric zones around the line of peaks of the newer volcanos.

1. Soils of the peak and upper slopes - allophane and kaolin clay and some less well developed intermediate texture which are generally of moderate erodibility but usually found in steep slopes.

2. Poorly developed, mainly sandy and loamy soils on the deep ash and agglomerates of the lower glaces slopes. These are soils of generally high erodibility but in gentler slopes.

3. Soils over limestone. Light textured, of small extent, quite erodible.

The USLE Model

The USLE Model, recently modified as the RUSLE (Revised USLE) Model, but essentially containing the same elements, is described as:

I = f ( Ed. Er. S. SL. C. P )

where

Ed is erodibility of soil – i.e. how readily the specific soil erodes under rain action

Er is erosivity of rainfall– i.e. how powerful the rainfall of the site is as an eroding force

S is slope angle at the site– i.e. how strongly the force of gravity is acting on soil particles

SL is slope length above the site– i.e. how much acceleration downslope water will have

C is land cover– i.e. what kind of cover overlies the soil – crop, with much bare soil,

continuous vegetation, concrete etc

P is land use management practices; i.e. what drainage, or strip usage or ridging etc has been used by the land holder

The model was designed, clearly enough, for agriculture, and agriculture of a specific system, but the elements of the model appear to be adequate for wide employment, even though details of the model itself need modification for tropical conditions

The model provides reasonably reliable, that is reproducible, results for middle-latitude conditions at any specific site, like one farm field. In that case it will give an estimate of actual soil losses to be expected. In the case of tropical latitudes in general and the Leewards in particular it can only be used as a general guideline. Fito the factors involved when water erosion over an area is being considered. In our use of the concepts we have accepted the distributions of erodibility, erosivity, the effect of gravity, and the effects of land cover and land use management practices.

The Soils of Antigua - Some Important Characteristics

(from Soils of Antigua & Barbuda - I.D. Hill et. al) and annotations

| Soil No. | Soil Type | Altitude and Topography | Drainage through Soil | Major Factors Limiting Root Penetration | Erosion Hazard Under Arable Crops | 9/66 Land-Use | Special Soil Management Problems | Landslide Susceptibility Value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| z | Mamora Clay Loam | Gentle, below 100' | Rapid | Compacted subsoil | Moderate | Rough grazing. Arable (cotton) | Compact subsoil. | 0 |
| 9 | Hodges Clay | Gentle, below 25' | Moderate | Heavy clay subsoil | Slight | Rough grazing | Salinity. | 2 |
| 10 | Yorks Silty Clay | Gentle, below 50' | Slow | Negligible in surface horizons, water table and heavy at depth | Slight | Arable (cane). Rough grazing | Drainage. Difficult to cultivate this heavy clay soil. | 0 |
| 11 | Dickensons Sandy Clay Loam | Gentle, below 100' | Rapid | Negligible | Slight | Arable (cane, ground provisions). Rough grazing | Saline or saline sodic subsoil. | 0 |
| 12 | Blubber Valley Sandy Loam | Gentle, below 200' | Rapid | Negligible, but occasionally stony | Slight | Arable (cane, ground provisions) | Stoniness. | 0 |
| 14 | Falmouth Sandy Clay Loam | Gentle, below 100' | Rapid | Compacted colluvium at 12" | Slight | Rough grazing | Compacted saline or saline sodic subsoil. | 0 |
| 15 | Red Hill Sandy Clay Loam | Gentle, below 100' | Rapid | Occasionally compacted subsoil | Slight | Arable (cane, cotton). Rough grazing | Compact sodic subsoil. Stoniness. | 0 |
| 16 | Belvedere Clay | Gentle, below 50' | Moderate to slow | Negligible in surface horizons. Water table at depth | Slight | Arable (cane) | Drainage. Difficult to cultivate this heavy clay soil. | 0 |
| 17 | Bendals Clay | Gentle, below 50' | Slow | Negligible in surface horizons. Water table and heavy at depth. | Slight | Arable (cane, food crops) | Drainage. Difficult to cultivate this heavy clay soil. | 0 |
| 18 | Yeamans Silty Clay | Gentle, below 75' | Slow | Negligible in surface horizons. Water table and heavy at depth | Slight | Arable (cane) | Drainage. Difficult to cultivate this heavy clay soil. | 0 |
| 20 | Wetherill Clay Loam | Moderate to steep, below 400' | Rapid | Parent material at 6 12" | Moderate to severe | Scrub | Shallowness. High CaCO3 content. Erodibility. | 1 |
| 21 | Fitches Clay | Gentle to moderate, below 200' | Rapid | Negligible | Moderate | Arable (cane, cotton, food crops). Pasture, scrub | High CaCO3 content. | 2 |
| 22 | Langfords Clay | Gentle to moderate, below 200' | Rapid to moderate | Negligible | Slight | Arable (cane, cotton, food crops) | High CaCO3 content. Heavy soil, difficult to cultivate. | 2 |
| 23 | Boon Sandy Loam | Gentle, below 100' | Rapid | Parent material at 6 - 12" | Moderate to severe | Rough grazing. Scrub | Shallowness. High CaCO3 | 0 |
| 24 | Elliots Clay | Gentle to moderate, 100 - 200' | Rapid to moderate | Parent material at 6 - 12" | Moderate to severe | Rough grazing. Scrub | Shallowness. Erodibility | 0 |
| 25 | Belmont Clay | Moderate, below 100' | Rapid | Parent material | Moderate | Scrub. Rough grazing | Shallowness. Stoniness. | 2 |
| 40 | Gunthorpes Clay | Gentle, below 100' | Moderate to poor | Highly calcareous or saline subsoil, that is often compacted | Slight | Arable (cane). Pasture and rough grazing | Heavy soil, difficult to cultivate. Saline and/or calcareous subsoil. | 2 |
| 41 | Osbourn Clay | Gentle, below 50' | Poor | Salinity and water table | Slight | Rough grazing | Salinity. Drainage. | 0 |
| 42 | Parham Clay Loam | Gentle, below 25' | Slow | Salinity and compacted gravelly horizon at 12" | Slight | Scrub. Rough grazing | Salinity. | 0 |
| 43 | Lindsey Clay | Gentle, below 300' | Moderate to\ poor | Highly calcareous subsoil or weathered parent material | Slight | Arable (cane), cotton, food crops. Rough grazing | Heavy soil, difficult to cultivate. Calcareous subsoil. | 2 |
| 50 | Clarehall Clay Loam | Gentle to moderate, below 100' | Rapid | Parent material close to surface | Slight | Pasture. Scrub | Stoniness, shallowness. | 0 |
| 60 | Ottos Clay | Moderate, below 400' | Moderate | Compacted subsoil | Slight to moderate | Arable (cane, food crops). Pasture and rough grazing. Scrub | Heavy soil, difficult to cultivate. Saline or saline sodic subsoil. | 2 |
| 61 | Tomlinsons Clay | Gentle, below 100' | Moderate | Heavy compacted subsoil or weathered parent material | Slight | Arable (cane). Pasture | Heavy soil, difficult to cultivate. Saline or saline sodic subsoil. | 2 |
| 62 | Ottos Clay (Stony phase) | Moderate, below 400' | Moderate | Variable amounts of stones and boulders, sometimes forming an impenetrable pavement | Moderate | Arable (cane, food crops). Rough grazing. Where stones become abundant only rough grazing | Stoniness. Saline or saline sodic subsoil. | 2 |
| 70 | St. Clair Clay | Moderate to steep, below 300' | Rapid | Parent material close to surface | Moderate to severe | Arable (food crops). Rough grazing. Scrub | Erodibility. Shallowness. Stoniness. | 2 |
| 73 | Isaac Clay Loam | Very steep, below 400' | Rapid | Parent material close to surface | Severe | Scrub | Erodibility. Shallowness. Stoniness. | 2 |
| 74 | Shirley Loam | Moderate to steep, below 500' | Rapid | Parent material close to surface | Moderate to severe | Rough grazing. Scrub, Arable (cotton) | Erodibility. Shallowness. Stoniness. | 2 |
| 75 | Picacilly Clay Loam | Moderate to steep, below 400' | Rapid | Parent material close to surface | Moderate to severe | Rough grazing. Scrub | Erodibility. Shallowness. Stoniness. | 2 |
| 76 | Indian Creek Loam | Steep, below 600' | Rapid | Parent material close to surface | Moderate to severe | Rough grazing. Scrub. Arable (cotton). | Erodibility. Shallowness. Stoniness. | 2 |
| 77 | Liberta Clay Loam | Moderate to steep, below 700' | Rapid | Parent material close to surface | Moderate to severe | Arable (cane, food crops). Rough grazing. Scrub | Erodibility. Shallowness. Stoniness. | 2 |
| 80 | Fry Clay Loam | Moderate to steep, below 1,000' | Rapid | Parent material close to surface | Moderate to severe | Arable (food crops). Rough grazing. Scrub | Erodibility. Shallowness. Stoniness. | -1 |
| 81 | Spring Hill Loam | Very steep, below 1,000' | Very rapid | Parent material close to surface | Very severe | Scrub | Erodibility. | 1 |
| 90 | Monteros Clay Loam | Moderate to very steep, up to 1,300' | Rapid | Parent material close to surface | Moderate to severe | Arable (food crops). Rough grazing. Scrub | Erodibility. Shallowness. Stoniness. | 2 |

Some Important Characteristics of the Soil Mapping Units of St. Kitts.

( from Lang and Carroll with annotations)

| Map Symbol | Soil | Dominant Slope Range | Drainage Through Soil | Any High Water Table | Moisture Supplying Capacity | Root Limiting Layer | Erosion Hazard | General Fertility | Any Special Management Problems | Stones-Boulders | Landslide Susceptibility Value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | Cockleshell Bay Loam Sand Frigate Bay Loamy Sand | 0-2° | Very rapid | - | Very low | Nil | Danger of wind erosion if clean cultivated | Low | Low fertility. Windblast | Nil. | 0 |
| 3 | Bird Rock Loamy Sand | 2 - 10° | Rapid | - | Low | Nil | Gullying on steeper slopes | Moderate | Moisture conservation. Stoniness | Generally stony and bouldery. | 1 |
| 4 | Peninsula Loamy Sand | 0 - 2° | Rapid, but slight impedance4 below C. | - | Low | Cemented sub-surface horizon | Very low | Moderately low | Moisture conservation. Salinity. | Few stones or boulders. | 2 |
| 5 | Shadwell Loamy Sand | 0 - 10° | Rapid | - | Low | Nil | Gullying on steeper slopes | Moderately high | Moisture conservation | Variable, not very stony. | 1 |
| 6 | Golden Rock Loamy Sand and Sandy Loam | 0 - 5° | Rapid, but often impeded in subsurface horizons | - | Low to moderate | Usually a compact subsurface horizon | Low | Moderately high | Moisture conservation. Prevention of “packing” in sub-surface horizons | Relatively few stones or boulders. | 2 |
| 7 | Ogee’s Sandy Loam | 5 - 10° | Rapid | - | Low | Nil | Liable to both sheetwash and gullying | Moderately high | Erosion | Some stones and boulders. | 1 |
| 8 | Sandy Bay Loamy Sand and Sandy Loam | 0 - 5° | Rapid | - | Low | Nil | Moderately low, tends to gully | Moderately high | Moisture conservation | Very variable, often very bouldery. | 1 |
| 9 | Harris Loamy Fine Sand | 5 - 20° | Rapid | - | Low | Nil | Much soil lost by creep | Moderately low | Erosion. Stoniness | Stony and bouldery. | 1 |
| 10 | Pump Loamy Fine Sand | 5 - 20° | Rapid | - | Low | Very fine sandy sub-surface horizons often “pack” | Easily eroded due to light texture | Moderately high | Moisture conservation. Erosion. Cultivation | Usually slightly stony and bouldery. | 2 |
| 11 | Brotherson’s Loamy Sand | 2 - 10° | Very rapid | - | Very low | Nil | Low | Moderate | “Dry” soil | Usually not very stony. | 1 |
| 12 | Lavington’s Loamy Fine Sand and Fine Loam | 5 - 10° | Rapid | - | Low | Nil | Low, an accumulating soil | Moderate | Moisture conservation | Relatively few stones or boulders. | 0 |
| 13 | Vambelle Silt Loam and Silt | 5 - 20° | Moderately rapid, often limited by impervious layer towards base of soil | - | Low | Cemented subsurface horizons | Easily deflated. Often severely gullied | Moderately high | Erosion. Cemented subsurface horizons | Often stony. | 2 |
| 14 | Estridge’s Loam and Sandy Loam | 2 - 10° | Moderately rapid |  | Moderate to low | Generally base of soil | Topsoil usually lost by sheet erosion | Moderate | Shallowness. Erosion | usually few stones | 2 |
| 15 | Mansion Loam | 5 - 20° | Moderately rapid | - | Moderate | Generally base of soil | Topsoil usually lost by sheet erosion | Moderate | Rehabilitation and conservation of topsoil | Usually few stones. | 2 |
| 16 | Durprote Loam | 5 - 25° | Moderately rapid | - | Moderate | Generally base of soil | Topsoil usually lost by sheet erosion | Moderately high | Rehabilitation and conservation of topsoil | Usually few stones. | 2 |
| 17 | Parson’s Loam | 5 - 20° | Moderately rapid | - | Moderate | Generally base of soil | Topsoil usually lost by sheet erosion | Moderately high | Rehabilitation and conservation of topsoil | Usually some stones or boulders. | 2 |
| 18 | Ottley’s Level Loam | 10 - 25° | Moderately rapid | - | Moderate to low | Generally base of soil | Topsoil usually lost by sheet erosion | Moderately high | Rehabilitation and conservation of topsoil | Usually some stones or boulders | 2 |
| 19 | Whites Clay Loam | 5 - 20° | Moderately rapid | - | Moderate | Increasing loss of structure in deeper subsoils | Topsoil usually lost by sheet erosion | Moderate | Rehabilitation and conservation of topsoil | Not appreciably stony or bouldery. | 2 |
| 20 | Wingfield Level Clay Loam | 5 - 25° | Moderately rapid | - | Moderate | Increasing loss of structure in deeper subsoils | Topsoil usually lost by sheet erosion | Moderate | Rehabilitation and conservation of topsoil | Not appreciably stony or bouldery | 2 |
| 21 | Fahie’s Clay Loam | 10 - 20° | Moderate | - | Moderate | Deeper subsoil is very compact | Topsoil usually lost by sheet erosion | Moderate | Rehabilitation and conservation of topsoil | Not appreciably stony or bouldery | 2 |
| 22 | Phillips. Level Silt Loam | 5 - 25° | Moderately rapid | - | Moderate | Structure lost in deeper subsoil | Easily eroded if plant cover removed | Moderate | Conservation of topsoil. Fairly low nutrient content | Not appreciably stony or bouldery | 2 |
| 23 | Mount Misery Silt Loam | 10 - 20° | Rapid | - | Moderate to low | Nil | Very liable to sheetwash erosion | Moderately low | Conservation of topsoil | Slightly stony. | 2 |
| 24 | Peaks Silt Loam | 15 -35° | Moderately rapid | - | Moderate | Generally base of soil | Only moderate danger under present vegetation | Moderate | Steepness of slopes. Difficulty of access | Some stones and boulders. | 2 |
| 25 | Jack in the Box Clay Loam | 5 - 20° | Moderately slow. Fairly impervious clay horizon | Imperfect. Surface puddling | Moderately high | Heavy clay subsoil | Topsoil easily lost when clean cultivated | Moderate | Cultivation. Preservation of subsurface structure | Few stones or boulders. | 2(4) |
| 26 | Bayford’s Loam | 2 - 10° | Usually moderate, may be impeded | Imperfect. Surface puddling | Moderate | Often a subsurface cemented horizon | Some sheetwash on slopes over 5° | Moderately high | Prevention of subsoil “panning” | Not very stony. | 2(4) |
| 27 | Conaree Hills Loam | 2 - 10° | Moderate | - | Moderate | Solid parent material | Moderately low | Moderately low | Stoniness | Generally stony and bouldery. | 2 |
| 28 | Sir Timonthy’s Clay Loam | 15 - 35 | Moderate | - | Moderate to low | Solid parent material | Topsoil easily lost | Low | Shallow soil on steep slopes | Stony and bouldery with rock outcrops. | 2 |
| 29 | Monkey Hill | 5 - 20° | Moderate | - | Moderate | Usually vase of soil | Very liable to sheetwash | Moderate | Erosion on steep slopes. Bouldery nature | Stony and bouldery. | 2 |
| 30 | Salt Pond Sand and Loamy Sand | 0 - 2° | Rapid | Usually below 2' - 3' | Moderate, but water usually brackish | Nil | Very low | Low | Low fertility. Salinity | Nil. | 0 |
| 31 | Muddy Pond silty Clay and Sandy Clay | 0 - 2° | Slow | At 12' - 18' | Moderately high, but water usually brackish | Poorly drained soil below 6" | Very low | Very low | Drainage. Salinity | Almost stone free. | 0 |
| 32 | Friar’s Bay Clay and Silty Clay | 0 - 2° | Very slow | Usually at 6' or less | High, brackish water | Wet, saline soil | Very low | Very low | Excessive salinity | Nil. | 0 |

Ratings of soil characteristics in formulating water (K’) and wind (I’) erodibility indices

First step. Read a general rating on texture class, with assumed 2% organic matter, coarse or weak structure and rapid drainage

| mode class | (C)S | L(C)S | (C)SL | (C)SC | (C)SCL | C | CL | L | FS/SiC | FS/SiCL | FS/SiL | LFS/Si | Si |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Water Initial K’ | 12 | 15 | 19 | 12 | 12 | 12 | 19 | 26 | 22 | 27 | 47 | 47 | 47 |
| Wind Initial I’ | 10 | 12 | 14 | 7 | 10 | 10 | 14 | 21 | 30 | 35 | 50 | 60 | 60 |

Second step. Modify the general rating using the modifiers additively

| Characteristic | K’ /I’ | -6 | -5 | -4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Drainage | K’ |  |  |  |  |  | veryrapid | rapid | rapid -mod | mod | mod-slow | slow | very slow |
|  | I’ |  |  |  |  | very slow | slow | mod - slow | mod | mod -rapid | rapid | very rapid |  |
| Depth | K’ |  |  |  |  |  |  | >4' |  | 2- 4' |  | 1-2' | <1 |
| Stones | K’ |  |  |  |  | yes |  | no |  |  |  |  |  |
|  | I’ |  |  | yes |  |  |  | no |  |  |  |  |  |
| Clay type | K’ | Al |  |  |  | MCa |  | X,MNa |  |  | K |  |  |
| Ped typeand size | K’ |  |  | cr, gr v. fine | cr, gr fine | cr, grmed | ab,sab £ med | other |  |  |  |  |  |
|  | I’ |  |  |  |  |  |  | other | cr, gr fine |  | cr,grv.fine |  |  |
| Pedstrength | K’+I’ |  |  |  |  | strong | mod- strong | mod | mod-weak | weak |  |  |  |
| Organic matter % | K’ | >6 |  | 5-6 |  | 3-4 |  | 2 |  |  |  |  |  |

Antigua & Barbuda Data for K & I Calculation

| Soil No. | Texture | Drainage 2 | PM | Depth | Present erosion | Structure (Type, Size, Strength) | Organic Matter |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | XCL.Ca/Na | I | Alluvium | >4 | 1 | SAB, M, S | 6 |
| 9 | MC.Ca.Na | I | Clay/sandstone | 36982 | 1 | AB, C, S | 5 |
| 10 | XSiC.Na | I | Alluvium | >4 | 1 | SAB, M, M | 3 |
| 11 | XSCL | I | Alluvium | >4 | 1 | SAB, M, M | 2 |
| 12 | XSL | G | Alluvium | >4 | 1 | SAB, M, M | 7 |
| 14 | XCL (Na) | G-I | Colluvium | >4 | 1 | SAB, M, M | 4 |
| 15 | XSCL.Na | G | Alluvium | >4 | 1 | SAB, M, S | 2 |
| 16 | XC.Ca | I | Alluvium | >4 | - | AB, M, S | 4 |
| 17 | XC.Ca.Mg | I | Alluvium | >4 | 0 | SAB, M, M | 8 |
| 18 | XSiC.Na | I | Alluvium | >4 | 1 | C, M, M | 2 |
| 20 | (ST)GriC | G | Limestone | <1 | 2 | C, F, S | 2 |
| 21 | KC.Ca | G | Marl | >4 | 1 | C, M, M | 5 |
| 22 | (ST)MC.Ca | G | Marl | 36982 | 1 | C, F, W | 6 |
| 23 | XSL | G | Calc Sandstone | <1 | 3 | C, M, W | 2 |
| 24 | MC | G | Firestone | <1 | 1 | SAB, M, S | 3 |
| 25 | XC.Ca | G | Limestone | 36982 | 2 | C, M, S | 2 |
| 40 | MC.Ca | I | Clay | >4 | 1 | SAB, M, M | 4 |
| 41 | XC.Na | VS | Clay | >4 | 0 | SAB, F, M | 4 |
| 42 | MC.Na | I | Gravel/clay | >4 | 1 | C, F, M | 7 |
| 43 | MC.Ca/Na | I | Clay | >4 | 1 | C, F, M | 4 |
| 50 | StMCL(Ca) | I | CG gravels | 36922 | 1 | C, M, M | 3 |
| 60 | MC.Ca/Na | I | Clay/tuff | >4 | 1 | C, M, S | 3 |
| 61 | MC.Ca/Na | I | Clay/volcanic sand | >4 | 1 | SAB, M, M | 5 |
| 62 | (ST)MC.Ca/Na | I | Clay/tuff | 36982 | 1 | C, F, S | 6 |
| 70 | XC.Ca | G | Tuff | 36922 | 2 | C, M, S | 3 |
| 73 | KCL.Ca | G | Argillite (tuff) | 36922 | 3 | SAB, M, M | 6 |
| 74 | (ST)XL.Ca | G | Tuff | 36982 | 2 | C, F, W | 4 |
| 75 | (ST)L.Na | G | Tuff | 36922 | 3 | AB, C, S | 4 |
| 76 | (ST2)KL.Mg.Na | G | Tuff | 36922 | 3 | C, M, M | 5 |
| 77 | (ST)CL.Ca | G | Tuff | <1 | 2 | C, M, M | 3 |
| 80 | MC.Ca/Mg | G | Andesite | <1 | 2 | AB, M, S | 4 |
| 81 | (ST)L | G | Dacite | <1 | 4 | C, M, M | 4 |
| 90 | XCL.Ca.Mg | G | Agglomerate | 36922 | 2 | C, M, M | 4 |

Antigua & Barbuda K & I Calculation

| Soil No. | Water (K) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Wind (I) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TE | ST | CL | DR | DE | TT | TS | OM | Total | TE | ST | DR | TT | TS | Total |
| 8 | 19 | 0 | 0 | 1 | 0 | -1 | -2 | -4 | 13 | 14 | 0 | 2 | 0 | -2 | 14 |
| 9 | 12 | 0 | -2 | 1 | 2 | 0 | -2 | -4 | 7 | 10 | 0 | 2 | 0 | -2 | 10 |
| 10 | 22 | 0 | 0 | 1 | 0 | -1 | 0 | -2 | 20 | 30 | 0 | 2 | 0 | 0 | 32 |
| 11 | 12 | 0 | 0 | 1 | 0 | -1 | 0 | 0 | 12 | 10 | 0 | 2 | -1 | 0 | 11 |
| 12 | 19 | 0 | 0 | 0 | 0 | -1 | 0 | -6 | 12 | 14 | 0 | 3 | -1 | 0 | 16 |
| 14 | 19 | 0 | 0 | 1 | 0 | -1 | 0 | -2 | 17 | 14 | 0 | 2 | -1 | 0 | 15 |
| 15 | 12 | 0 | 0 | 0 | 0 | -1 | -2 | 0 | 9 | 14 | 0 | 3 | 0 | -2 | 15 |
| 16 | 12 | 0 | 0 | 1 | 0 | -1 | -2 | -2 | 8 | 10 | 0 | 2 | -1 | -2 | 9 |
| 17 | 12 | 0 | 0 | 1 | 0 | -1 | 0 | -6 | 6 | 10 | 0 | 2 | -1 | 0 | 11 |
| 18 | 22 | 0 | 0 | 1 | 0 | -2 | 0 | 0 | 21 | 30 | 0 | 2 | 0 | 0 | 32 |
| 20 | 12 | -2 | 0 | 0 | 5 | -4 | -2 | 0 | 9 | 10 | -4 | 3 | 1 | -2 | 8 |
| 21 | 12 | 0 | 0 | 0 | 0 | -2 | 0 | -4 | 6 | 10 | 0 | 3 | -2 | 0 | 11 |
| 22 | 12 | -2 | -2 | 0 | 2 | -3 | 2 | -5 | 4 | 10 | -4 | 3 | -3 | 2 | 8 |
| 23 | 19 | 0 | 0 | 0 | 5 | -2 | 2 | 0 | 24 | 14 | 0 | 3 | -2 | 2 | 17 |
| 24 | 12 | 0 | 0 | 0 | 5 | -1 | -2 | -2 | 12 | 10 | 0 | 3 | -1 | -2 | 10 |
| 25 | 12 | 0 | 0 | 0 | 2 | -2 | -2 | 0 | 10 | 10 | 0 | 3 | -2 | -2 | 9 |
| 26 | 12 | 0 | -2 | 1 | 4 | -3 | -2 | -2 | 8 | 10 | 0 | 2 | 1 | -2 | 11 |
| 27 | 27 | 0 | -2 | 1 | 5 | -4 | -2 | -2 | 23 | 35 | 0 | 2 | 3 | -2 | 38 |
| 28 | 12 | 0 | 3 | 0 | 4 | -3 | -2 | -2 | 12 | 10 | 0 | 3 | 1 | -2 | 12 |
| 40 | 12 | 0 | -2 | 1 | 0 | -1 | 0 | -2 | 8 | 10 | 0 | 2 | -1 | 0 | 11 |
| 41 | 12 | 0 | 0 | 5 | 0 | -1 | 0 | -2 | 14 | 10 | 0 | -2 | 0 | 0 | 8 |
| 42 | 19 | 0 | 0 | 1 | 0 | -4 | 0 | -6 | 10 | 14 | 0 | 2 | 1 | 0 | 17 |
| 43 | 12 | 0 | -2 | 1 | 0 | -4 | 0 | -2 | 5 | 10 | 0 | 2 | 1 | 0 | 13 |
| 50 | 19 | -2 | -2 | 1 | 4 | -2 | 0 | -2 | 16 | 14 | -4 | 2 | -2 | 0 | 10 |
| 60 | 12 | 0 | -2 | 1 | 0 | -2 | -2 | -2 | 5 | 10 | 0 | 2 | 0 | -2 | 10 |
| 61 | 12 | 0 | -2 | 1 | 0 | -1 | 0 | -4 | 6 | 10 | 0 | 2 | 0 | 0 | 12 |
| 62 | 12 | -2 | -2 | 1 | 2 | -4 | -2 | -4 | 1 | 10 | -4 | 2 | 1 | -2 | 7 |
| 70 | 12 | 0 | -2 | 0 | 4 | -2 | -2 | -2 | 8 | 10 | 0 | 3 | 0 | -2 | 11 |
| 73 | 19 | 0 | 3 | 0 | 4 | -1 | 0 | -5 | 20 | 21 | 0 | 3 | -1 | 0 | 23 |
| 74 | 26 | -2 | 0 | 0 | 2 | -4 | 2 | -2 | 22 | 21 | -4 | 3 | 1 | 2 | 23 |
| 75 | 26 | -2 | 0 | 0 | 4 | 0 | -2 | -2 | 24 | 21 | -4 | 3 | 0 | -2 | 18 |
| 76 | 26 | -2 | 3 | 0 | 4 | -2 | 0 | -4 | 25 | 21 | -4 | 3 | -2 | 0 | 18 |
| 77 | 19 | -2 | 0 | 0 | 5 | -2 | 0 | -2 | 18 | 14 | -4 | 3 | -2 | 0 | 11 |
| 80 | 12 | 0 | -2 | 0 | 5 | -2 | -2 | -2 | 9 | 10 | 0 | 3 | -2 | -2 | 9 |
| 81 | 26 | -2 | 0 | 0 | 5 | -2 | 0 | -2 | 25 | 21 | -4 | 3 | 0 | 0 | 20 |
| 90 | 19 | 0 | 0 | 0 | 4 | -2 | 0 | -2 | 19 | 14 | 0 | 3 | 0 | 0 | 17 |
| 97 | 15 | 0 | 0 | -1 | 0 | 0 | 0 | 1 | 15 | 12 | 0 | 4 | 0 | 0 | 16 |

St. Kitts Data for K & I Calculation

| Soil No. | Texture | Drainage 2 | PM | Depth | Structure | Organic Matter |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | LS.Ca | E | Beach sand | >4 | G, F, VW | 4+ |
| 2 | LS.Ca | E | Beach sand | >4 | G, F, VW | 4+ |
| 3 | LS | G | Colluvium | >4 | AB, M, VW | 4+ |
| 4 | LS | G | Colluvium | >4 | SG | 1 |
| 5 | LS | G | Ash | >4 | G, F, W | 1 |
| 6 | LS | G | Ash | >4 | AB, M, M | 2 |
| 7 | SL | G | Ash + | >4 | AB, F, W | 1 |
| 8 | LS | G | Ash | >4 | G, F, W | 2 |
| 9 | LFS | G | Ash | >4 | G, F, W | 4 |
| 10 | LFS | G | Ash | >4 | G, F, W | 2 |
| 11 | LS | E | Colluvium/ash | >4 | G, F, W | 4 |
| 12 | LFS | G | Colluvium/ash | >4 | G, F, W | 2 |
| 13 | SiL | I | Cemented ash/tuff | 2-4 (+) | G, F, M | 2 |
| 14 | L | I | Agg tuff | 1-2 (+) | G, F, W | 4 |
| 15 | L | I | Agg tuff | 36982 | G, F, M | 2 |
| 16 | L | I | Agg tuff | 36982 | G, F, M | 1 |
| 17 | L | I | Tuff | 36982 | AB, M, M | 4 |
| 18 | L | I | Stony tuff | 36982 | AB, M, W | 3 |
| 19 | KCL | I | Tuff | 2-4(+) | C, F, S | 2 |
| 20 | KCL | I | Agg tuff | 2-4(+) | AB, M, M | 3 |
| 21 | KCL | I | Tuff | >4 | C, F, M | 4 |
| 22 | K-Al.SiL | I | Stony tuff | 2-4(+) | C, F, W | 5 |
| 23 | K-Al.SiL | G | Sandy ash | 2-4(+) | C, F, W | 6 |
| 24 | AL.SiL | I | Agg tuff/ash | 36982 | C, F, W | 30 |
| 25 | SCL | I-P | Agglomerate | 36982 | SAB, M, M | 5 |
| 26 | SL | I | Tuffy agglomerate | 36922 | AB, M, W | 3 |
| 27 | SL | I | Andesite | 36922 | AB, M, W | 3 |
| 28 | SCL | I | Agglomerate | <1 | AB, M, W | 2 |
| 29 | SCL | I | Agglomerate | 36922 | SAB, M, M | 2 |
| 30 | M.LS.Na | G | Alluvium | >4 | SG | 1 |
| 31 | M.SiC.Na | P | Alluvium | >4 | AB, C, S | 2 |
| 32 | M.SiC.Na | P | Alluvium | >4 | SG | 1 |

St. Kitts K and I calculation

| Soil No. | Water (K) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Wind (I) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TE | ST | CL | DR | DE | TT | TS | OM | TOTAL | TE | ST | DR | TT | TS | TOTAL |
| 1 | 15 | 0 | 0 | -1 | 0 | -3 | 2 | -2 | 11 | 12 | 0 | 4 | 1 | 2 | 19 |
| 2 | 15 | 0 | 0 | -1 | 0 | -3 | 2 | -2 | 11 | 12 | 0 | 4 | 1 | 2 | 19 |
| 3 | 15 | 0 | 0 | 0 | 0 | -1 | 2 | -2 | 14 | 12 | 0 | 3 | 0 | 2 | 17 |
| 4 | 15 | 0 | 0 | 0 | 0 | -3 |  | 1 | 13 | 12 | 0 | 3 |  | 2 | 17 |
| 5 | 15 | 0 | 0 | 0 | 0 | -3 | 2 | 1 | 15 | 12 | 0 | 3 | 1 | 2 | 18 |
| 6 | 15 | 0 | 0 | 0 | 0 | -1 | 0 | 0 | 14 | 12 | 0 | 3 | 0 | 0 | 15 |
| 7 | 19 | 0 | 0 | 0 | 0 | -1 | 0 | 1 | 19 | 14 | 0 | 3 | 0 | 1 | 18 |
| 8 | 15 | 0 | 0 | 0 | 0 | -3 | 2 | 0 | 14 | 12 | 0 | 3 | 1 | 2 | 18 |
| 9 | 47 | 0 | 0 | 0 | 0 | -3 | 2 | -2 | 44 | 60 | 0 | 3 | 1 | 2 | 66 |
| 10 | 47 | 0 | 0 | 0 | 0 | -3 | 2 | 0 | 46 | 60 | 0 | 3 | 1 | 2 | 66 |
| 11 | 15 | 0 | 0 | -1 | 0 | -3 | 2 | -2 | 11 | 12 | 0 | 4 | 1 | 2 | 19 |
| 12 | 47 | 0 | 0 | 0 | 0 | -3 | 2 | 0 | 46 | 60 | 0 | 3 | 1 | 2 | 66 |
| 13 | 47 | 0 | 0 | 1 | 2 | -3 | 0 | 0 | 47 | 60 | 0 | 2 | 1 | 0 | 63 |
| 14 | 26 | 0 | 0 | 1 | 4 | -3 | 2 | -2 | 28 | 21 | 0 | 2 | 1 | 2 | 26 |
| 15 | 26 | 0 | 0 | 1 | 2 | -3 | 0 | 0 | 26 | 21 | 0 | 2 | 1 | 0 | 24 |
| 16 | 26 | 0 | 0 | 1 | 2 | -3 | 0 | 1 | 27 | 21 | 0 | 2 | 1 | 0 | 24 |
| 17 | 26 | 0 | 0 | 1 | 2 | -1 | 0 | -2 | 26 | 21 | 0 | 2 | 0 | 0 | 23 |
| 18 | 26 | 0 | 0 | 1 | 2 | -1 | 2 | -2 | 28 | 21 | 0 | 2 | 0 | 2 | 25 |
| 19 | 19 | 0 | 3 | 1 | 2 | -3 | -2 | 0 | 20 | 14 | 0 | 2 | 1 | -2 | 15 |
| 20 | 19 | 0 | 3 | 1 | 2 | -1 | 0 | -2 | 22 | 14 | 0 | 2 | 0 | 0 | 16 |
| 21 | 19 | 0 | 3 | 1 | 0 | -3 | 0 | -2 | 18 | 14 | 0 | 2 | 1 | 0 | 17 |
| 22 | 47 | 0 | -6 | 1 | 2 | -3 | 2 | -4 | 39 | 50 | 0 | 2 | 1 | 2 | 55 |
| 23 | 47 | 0 | 0 | 0 | 2 | -3 | 2 | -4 | 44 | 50 | 0 | 3 | 1 | 2 | 56 |
| 24 | 12 | 0 | -6 | 1 | 2 | -3 | 2 | -4 | 4 | 50 | 0 | 2 | 1 | 2 | 55 |
| 25 | 12 | 0 | 0 | 2 | 2 | -1 | 0 | -4 | 11 | 10 | 0 | 1 | 0 | 0 | 11 |
| 26 | 19 | 0 | 0 | 1 | 4 | -1 | 2 | -2 | 23 | 14 | 0 | 2 | 0 | 2 | 18 |
| 27 | 19 | 0 | 0 | 1 | 4 | -1 | 2 | -2 | 23 | 14 | 0 | 2 | 0 | 2 | 18 |
| 28 | 12 | 0 | 0 | 1 | 5 | -1 | 2 | 0 | 19 | 10 | 0 | 2 | 0 | 2 | 14 |
| 29 | 12 | 0 | 0 | 1 | 4 | -1 | 0 | 0 | 16 | 10 | 0 | 2 | 0 | 0 | 12 |
| 30 | 15 | 0 | 0 | 0 | 0 |  | -2 | 1 | 14 | 12 | 0 | 3 |  | 2 | 17 |
| 31 | 22 | 0 | 0 | 4 | 0 | -1 | -2 | 0 | 23 | 30 | 0 | -1 | 0 | -2 | 27 |
| 32 | 22 | 0 | 0 | 4 | 0 |  | -2 | 1 | 25 | 30 | 0 | -1 |  | 2 | 31 |

Slope Factor Calculation

| Mean slope (degrees) | 100 (tan x)1.55 | Slope value used (water erosion) | Slope value used (landslides) |
| --- | --- | --- | --- |
| 1 | 0.188 | 0.2 | 0.2 |
| 1.5 | 0.353 | 0.3 | 0.3 |
| 2.5 | 0.78 | 0.8 | 0.8 |
| 3 | 1.035 | 1 | 1 |
| 3.5 | 1.315 | 1.3 | 1.3 |
| 6 | 3.044 | 3 | 3 |
| 6.5 | 3.45 | 3.5 | 3.5 |
| 7.5 | 4.316 | 4.3 | 4.3 |
| 12.5 | 9.681 | 9.7 | 9.7 |
| 13.5 | 10.953 | 10.9 | 10.9 |
| 15 | 12.98 | 13 | 13 |
| 20 | 20.876 | 20.9 | 20.9 |
| 22.5 | 25.509 | 25.5 | 25.5 |
| 25 | 30.65 | 30.7 | 30.7 |
| 27.5 | 36.35 | 36.3 | 36.3 |
| 32.5 | 49.715 | 49.7 | 99.4 |
| 33.5 | 52.748 | 52.7 | 105.4 |
| 35 | 57.563 | 57.6 | 115.2 |

Antigua Geology: Gullying Susceptibility & Landslide Type

|  | Code | Geology | Gullying Susceptibility | Landslide Type |
| --- | --- | --- | --- | --- |
| 1 | CC | Corbison Chert Series | 1 | 1 |
| 2 | T | Tuffs | 1 | 1 |
| 3 | O | Other igneous | 0 | 2 |
| 4 | B | Basalt | 0 | 2 |
| 5 | L | Limestone | 0 | 2 |
| 6 | U | Unconsolidated | 1 | 1 |
| 7 | A | Agglomerates | 1 | 1 |
| 8 | C | Coarse agglomerates | 1 | 1 |
| 9 | AF | Antigua Formation | -1 | 2 |
| 10 | CO | Conglomerate | 0 | 1 |

Gullying Susceptibility: 	0 = Improbable 			1 = Probable

Landslide Type: 	1 = Soil and weathered surface	2 = Soil, weathered surface and rock fall

St. Kitts Geology: Gullying Susceptibility & Landslide Type

|  | Code | Geology | Gullying Susceptibility | Landslide Type |
| --- | --- | --- | --- | --- |
| 1 | BD | Beach deposits | 1 | 0 |
| 2 | MD | Marsh deposits | 1 | 0 |
| 3 | BH | Brimstone Hill andesite | 0 | 2 |
| 4 | OM | Ottleys Mt. andesite | 0 | 2 |
| 5 | SP | Sandy Point Hill (andesite dome/plug) | 0 | 2 |
| 6 | MH | Monkey Hill(andesite dome/plug) | 0 | 2 |
| 7 | BR | Black Rocks basalt | 0 | 2 |
| 8 | MM | Later tuffs, etc. of Mt. Misery | 1 | 1 |
| 9 | BT | Basseterre tuffs | 1 | 1 |
| 10 | BL | Brimstone Hill limestone | 0 | 2 |
| 11 | GG | Godwin ghaut limestone | 0 | 2 |
| 12 | DM | Deposits of Mt. Misery Centre (ash + agglomerate) | 1 | 1 |
| 13 | MR | Middle Range Centre (andesite) | 0 | 2 |
| 14 | SR | Southeast Range/Olivees Mt. Centres (andesite) | 0 | 2 |
| 15 | OV | Older volcanics (andesite + coarse block agglomerates) | 0 | 2 |
| 16 | SP | Salt Pond | 0 | 0 |

Gullying Susceptibility: 	0 = Improbable 			1 = Probable

Landslide Type: 	1 = Soil and weathered surface	2 = soil, weathered surface and rock fall

Land cover/vegetation factor for St. Kitts

|  |  |  |  |  | Water Entry (infiltration) Reduction | Water Erosion Factor | Landslide Factor |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | F.R. | Forest reserve | 0 | 0.01 | 0.1 |
| 7 | W. | Woodland | 0 | 0.1 | 0.2 |
| 5 | SC | Sugar cane | 10 | 5 | 1 |
| 2 | CF | Crop farming | 50 | 60 | 1 |
| 3 | Q | Quarry | 75 | 0 | 1 |
| 4 | SB | Scrub | 20 | 5 | 1 |
| 6 | S | Built up/ playgrounds etc. | 1001 | 0 (var)1 | 1 |

Note

1. The land cover map used was a planning document and includes projected as well as actual building developments.

2. Reduction of water entry into soil by 100% and erosion factor to 0, only where land is built over - but this will include contracted playing fields.

St. Kitts: Infiltration Reduction Estimate

|  | Area Count Under Land Types |  |  |  |  |  |  |  |  |  |  |  |  | Proportion of Area Under Land Types |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 10 | 20 | 50 | 75 | 100 | TOTAL | 0.1 | 0.2 | 0.5 | 1 | TOTAL |
| 1 | 4855 | 3622 | 1376 | 1454 | 0 | 2995 | 14302 | 0.0253 | 0.0192 | 0.05083 | 0.2094 | 0.3047 |
| 2 | 68 | 0 | 0 | 0 | 0 | 2803 | 2871 | 0 | 0 | 0 | 0.9763 | 0.9763 |
| 3 | 253 | 230 | 0 | 660 | 0 | 578 | 1721 | 0.0133 | 0 | 0.191749 | 0.3359 | 0.5409 |
| 4 | 3185 | 1692 | 306 | 2207 | 0 | 4259 | 11649 | 0.0145 | 0.005254 | 0.09473 | 0.3656 | 0.4801 |
| 5 | 3968 | 193 | 0 | 26 | 0 | 1779 | 5966 | 0.0032 | 0 | 0.0022 | 0.2982 | 0.3035 |
| 6 | 2538 | 1799 | 0 | 130 | 0 | 959 | 5426 | 0.0331 | 0 | 0.01198 | 0.1767 | 0.2218 |
| 7 | 3558 | 0 | 0 | 382 | 0 | 246 | 4186 | 0 | 0 | 0.04563 | 0.059 | 0.1044 |
| 8 | 1836 | 0 | 0 | 2219 | 0 | 1060 | 5115 | 0 | 0 | 0.216911 | 0.2072 | 0.4241 |
| 9 | 1780 | 321 | 0 | 282 | 0 | 2177 | 4560 | 0.007 | 0 | 0.03092 | 0.4774 | 0.5153 |
| 10 | 5362 | 4113 | 193 | 544 | 0 | 1346 | 11558 | 0.0355 | 0.00334 | 0.02353 | 0.1165 | 0.1789 |
| 11 | 3535 | 139 | 0 | 348 | 0 | 119 | 4141 | 0.0033 | 0 | 0.04202 | 0.029 | 0.074 |
| 12 | 8977 | 1263 | 0 | 292 | 0 | 538 | 11070 | 0.0114 | 0 | 0.01319 | 0.049 | 0.073 |
| 13 | 3338 | 486 | 0 | 0 | 0 | 496 | 4320 | 0.0112 | 0 | 0 | 0.1148 | 0.126 |
| 14 | 3651 | 601 | 0 | 1180 | 0 | 258 | 5690 | 0.0105 | 0 | 0.103691 | 0.045 | 0.1595 |
| 15 | 6 | 1018 | 0 | 123 | 0 | 275 | 1422 | 0.0715 | 0 | 0.04325 | 0.1934 | 0.3081 |
| 16 | 6841 | 1052 | 0 | 832 | 0 | 207 | 8932 | 0.0117 | 0 | 0.04657 | 0.023 | 0.081 |
| 17 | 396 | 73 | 0 | 562 | 0 | 2506 | 3537 | 0.002 | 0 | 0.07945 | 0.7085 | 0.7899 |
| 18 | 423 | 558 | 0 | 345 | 0 | 724 | 2050 | 0.0272 | 0 | 0.08415 | 0.3532 | 0.4645 |
| 19 | 1050 | 1277 | 0 | 261 | 0 | 162 | 2750 | 0.0464 | 0 | 0.04745 | 0.059 | 0.1527 |
| 20 | 4444 | 4653 | 751 | 1732 | 0 | 476 | 12056 | 0.0385 | 0.0124 | 0.07183 | 0.039 | 0.1622 |
| 21 | 13 | 5968 | 0 | 266 | 0 | 771 | 7018 | 0.085 | 0 | 0.01895 | 0.1099 | 0.2138 |
| 22 | 14 | 2805 | 0 | 0 | 0 | 419 | 3238 | 0.0866 | 0 | 0 | 0.1294 | 0.216 |
| 23 | 3119 | 2738 | 0 | 2457 | 0 | 14 | 8328 | 0.0328 | 0 | 0.1475 | 0 | 0.1819 |
| 24 | 0 | 1291 | 0 | 0 | 0 | 36 | 1327 | 0.0972 | 0 | 0 | 0.027 | 0.1243 |
| 25 | 772 | 2808 | 0 | 865 | 0 | 1245 | 5690 | 0.0493 | 0 | 0.07601 | 0.2188 | 0.3441 |
| 26 | 3649 | 2778 | 1810 | 907 | 0 | 3086 | 12230 | 0.0227 | 0.0295993 | 0.03708 | 0.2523 | 0.3417 |
| 27 | 487 | 10 | 2345 | 0 | 0 | 0 | 2842 | 0.0003 | 0.165 | 0 | 0 | 0.1653 |
| 28 | 1607 | 1086 | 2945 | 222 | 0 | 925 | 6785 | 0.016 | 0.0868 | 0.01636 | 0.1363 | 0.2554 |
| 29 | 1725 | 504 | 310 | 190 | 0 | 196 | 2925 | 0.0172 | 0.0211966 | 0.03248 | 0.067 | 0.1378 |
| 30 | 1415 | 834 | 0 | 284 | 0 | 102 | 2635 | 0.0316 | 0 | 0.05389 | 0.039 | 0.1242 |
| 31 | 471 | 1105 | 1014 | 906 | 0 | 33 | 3529 | 0.0313 | 0.0574 | 0.1283 | 0.01 | 0.2263 |
| 32 | 655 | 2167 | 1153 | 881 | 0 | 11 | 4867 | 0.0445 | 0.0473 | 0.09051 | 0 | 0.1845 |
| 33 | 78 | 0 | 715 | 234 | 0 | 84 | 1111 | 0 | 0.1287 | 0.1053 | 0.076 | 0.3096 |
| 34 | 777 | 3298 | 1793 | 0 | 0 | 9336 | 15204 | 0.0216 | 0.0235 | 0 | 0.614 | 0.6591 |
| 35 | 1416 | 1257 | 3 | 925 | 0 | 2 | 3603 | 0.0348 | 0.00017 | 0.1283 | 0 | 0.1638 |
| 36 | 1847 | 3767 | 1 | 1019 | 0 | 784 | 7418 | 0.0507 | 0 | 0.06868 | 0.1057 | 0.2251 |
| 37 | 7732 | 3516 | 77 | 1235 | 0 | 298 | 12858 | 0.0273 | 0.001198 | 0.04802 | 0.023 | 0.1 |
| 38 | 349 | 804 | 0 | 268 | 0 | 771 | 2192 | 0.0366 | 0 | 0.06113 | 0.3517 | 0.4494 |
| 39 | 3481 | 1476 | 0 | 2072 | 0 | 0 | 7029 | 0.0209 | 0 | 0.1473 | 0 | 0.1682 |
| 40 | 8871 | 1283 | 30879 | 257 | 188 | 4574 | 46052 | 0.0027 | 0.1341 | 0.0028 | 0.099 | 0.2389 |
| 41 | 195 | 0 | 334 | 184 | 0 | 441 | 1154 | 0 | 0.0578 | 0.07972 | 0.3821 | 0.5196 |
| 42 | 2206 | 2705 | 0 | 271 | 0 | 196 | 5378 | 0.0502 | 0 | 0.0252 | 0.036 | 0.1118 |
| 43 | 257 | 91 | 615 | 397 | 0 | 501 | 1861 | 0.0048 | 0.066 | 0.1066 | 0.2692 | 0.4466 |
| 44 | 90 | 2487 | 230 | 0 | 0 | 607 | 3414 | 0.0728 | 0.0134 | 0 | 0.1778 | 0.264 |
| 45 | 6610 | 2583 | 0 | 780 | 0 | 3972 | 13945 | 0.0185 | 0 | 0.02797 | 0.2848 | 0.3313 |
| 46 | 2250 | 52 | 0 | 1518 | 0 | 1130 | 4950 | 0.001 | 0 | 0.1533 | 0.2283 | 0.3825 |
| 47 | 696 | 5395 | 0 | 0 | 0 | 750 | 6841 | 0.0788 | 0 | 0 | 0.1096 | 0.1884 |
| 48 | 350 | 564 | 0 | 0 | 0 | 1112 | 2026 | 0.0278 | 0 | 0 | 0.5489 | 0.5766 |
| 49 | 242 | 1258 | 977 | 979 | 0 | 4759 | 8215 | 0.0153 | 0.0237 | 0.05959 | 0.5793 | 0.6778 |
| 50 | 249 | 1443 | 0 | 0 | 0 | 803 | 2495 | 0.0578 | 0 | 0 | 0.3218 | 0.3796 |
| 51 | 772 | 1288 | 0 | 0 | 0 | 1935 | 3995 | 0.0322 | 0 | 0 | 0.4844 | 0.5165 |
| 52 | 221 | 1519 | 0 | 0 | 0 | 445 | 2185 | 0.0695 | 0 | 0 | 0.2037 | 0.2731 |
| 53 | 761 | 1493 | 0 | 0 | 0 | 587 | 2841 | 0.0525 | 0 | 0 | 0.2066 | 0.2591 |
| 54 | 0 | 3354 | 5 | 299 | 0 | 33 | 3691 | 0.0908 | 0.00027 | 0.0405 | 0.01 | 0.1405 |
| 55 | 0 | 3025 | 0 | 0 | 0 | 81 | 3106 | 0.0973 | 0 | 0 | 0.026 | 0.1233 |
| 56 | 18 | 1433 | 0 | 510 | 0 | 42 | 2003 | 0.0715 | 0 | 0.1273 | 0.021 | 0.2197 |